Section 083613
Aluminum sectional Doors
Models 451, 452, 453, 464, K-AL

NOTE: Enable Microsoft Word option to display hidden text to view notes to specifier.

PART 1 GENERAL

Section Includes

Upward acting sectional aluminum doors.

Related Requirements

Section << 079200 - Joint Sealants>>:  Sealing joints between frames and adjacent construction.

Section << 087100 - Door Hardware>>:  Cylinder cores and keys.

Section << 260533.13 - Conduit for Electrical Systems>>:  Conduit from electric circuit to operator and from operator to control station, and empty conduit from control units to door operator.

Section << 260583 - Wiring Connections>>:  Power to disconnect.

Definitions

DSB:  Double Strength Billet.

SSB:  Single Strength Billet.

Reference Standards

ITS (DIR) - Directory of Listed Products; Current Edition.

UL (DIR) - Online Certifications Directory; Current Edition.

UL 325 - Standard for Door, Drapery, Gate, Louver, and Window Operators and Systems; Current Edition, Including All Revisions.

~~~~~ MASTER NOTE ~~~~~~
Do not request submittals if drawings sufficiently describe the products of this section or if proprietary specifying techniques are used. The review of submittals increases the possibility of unintended variations from contract documents, thereby increasing the design professional's liability.

- ~~~ END OF MASTER NOTE ~~~~

Submittals

Product Data:  Submit manufacturer's standard literature showing materials and details of construction and finish.<<  Include data on electrical operation.; or  None - N/A>>

Shop Drawings:  Indicate rough and actual opening dimensions, anchorage methods, hardware locations, and installation details.

Manufacturer's Installation Instructions:  Indicate installation sequence and installation, adjustment, and alignment procedures.

Manufacturer's qualification statement.

Installer's qualification statement.

Operation and Maintenance Data:  Indicate modes of operation, lubrication requirements and frequency, and periodic adjustments required.

Specimen warranty.

Quality Assurance

Manufacturer Qualifications:  Company specializing in performing work of this section with minimum of << 5 years; or  _____ years>> of<<  documented;    _____; or  None - N/A>> experience in fabrication and installation of security closures.

Installer Qualifications:  Company specializing in performing work of this section with minimum of << 3 years; or  _____ years>> of<<  documented;    _____; or  None - N/A>> experience and approved by manufacturer.

Warranty

Manufacturer Door-Only Warranty:  Provide manufacturer warranty for door assembly for duration indicated under individual doors. Complete forms in <<Owner>>'s name and register with manufacturer.

PART 2 PRODUCTS

Manufacturer

- ~~~~~ MASTER NOTE ~~~~~~
Founded in 1954, Wayne Dalton was built on a dedication to ingenuity and customer service. As our business grew, so did our commitment to expanding our product lines, leading the company to become one of the largest door manufacturers in the United States. Since the beginning, we have remained committed to being the garage door partner whose innovation drives dealer productivity and provides industry-leading garage door solutions. In continuing this innovation, Wayne Dalton offers products, not only through our dealer connections, but also through our retail partner, Lowe's. Wayne Dalton offers a broad selection of residential garage doors, as well as a complete line of doors for commercial and industrial applications. Our Commercial Sectional Doors are ideal for jobs that call for excellent thermal efficiency, have stringent wind load requirements, or simply need a high performing door designed for long life and superior strength. Our Commercial Rolling Doors include service doors, counter doors, fire doors, security grilles, security shutters and roll up sheet doors. In 2016, we expanded our commercial door category to include High Speed Doors. Wayne Dalton designed and built a new series of High-Speed Doors from the ground up. These doors feature patent-pending, high-cycle designs, and industry-leading safety features for both interior and exterior applications. With everything from residential garage doors to rolling doors to commercial sectional to high performance doors, Wayne Dalton’s breadth of product can provide a garage door solution for any application.   Contact Information: Wayne Dalton 2501 S State Hwy 121 Bus, Suite 200 Lewisville, Texas 75067 United States   Phone: (800) 827-3667 Email: info@wayne-dalton.com   Contact for Design Professional: Customer Service Phone: (800) 827-3667 Email: info@wayne-dalton.com   Online CEU Courses   Request Lunch & Learn   https://www.wayne-dalton.com/   Plant Location: Dalton, Ohio; Mt. Hope, Ohio, Pensacola, Florida; Centralia, Washington.    
~~~ END OF MASTER NOTE ~~~~

Wayne Dalton; www.wayne-dalton.com; 1 (800) 827-3667.

Substitutions: Not permitted.

Overhead Sectional Aluminum Doors

Aluminum Sectional Doors:  Wayne Dalton; << Model 451;  Model 452;  Model 453;  Model 464; or  Model K-AL>>.

Panels:  Aluminum construction and reinforcement.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum widths:
Models 451, 452, and 453: 22 feet 2 inches (6.77 m)
Model K-AL: 24 feet (7.32 m)
Model 464: 18 feet 2 inches (5.55 m)
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet _____ inches (_____ m)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum heights: 
Model 451: 16 feet 1 inch (4.91 m)
Model 452 and 464: 14 feet 2 inches (4.30 m)
Model 453: 20 feet 1 inch (6.13 m)
Model K-AL: 18 feet (5.49 m)
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet _____ inches (_____ m)>>.

Mounting:  Surface mounted on interior of building.

Opening Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

Closing Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

- ~~~~~ MASTER NOTE ~~~~~~U-factor values (Models 452, 453 and K-AL only; Models 451 and 464 do not carry a U-factor):Model 452: 1/2-inch insulated glass 0.86; 1/2-inch Low-E insulated glass 0.70.Model 453: 1/2-inch insulated glass 0.93; 1/2-inch Low-E insulated glass 0.85.Model K-AL: 1/2-inch insulated glass 0.90; 1/2-inch Low-E insulated glass 0.80.U-factors are based on independent testing and verified per ANSI/DASMA 105.~~~ END OF MASTER NOTE ~~~~

U-Factor (Models 452, 453 and K-AL only):  << Model 452: 0.86 (1/2-inch insulated glass) or 0.70 (1/2-inch Low-E insulated glass);  Model 453: 0.93 (1/2-inch insulated glass) or 0.85 (1/2-inch Low-E insulated glass);  Model K-AL: 0.90 (1/2-inch insulated glass) or 0.80 (1/2-inch Low-E insulated glass)>>.

- ~~~~~ MASTER NOTE ~~~~~~
R-value (RSI-value) for Model 453:
 With double strength billet (DSB) clear, tempered, or obscure glazing IGUs: R-value (RSI-value): 2.87 (0.51).
 With clear polycarbonate IGUs: R-value (RSI-value): 2.93 (0.52).
 With solar bronze IGUs: R-value (RSI-value): 3.17 (0.56).
 With Low-E coating IGUs: R-value (RSI-value): 3.43 (0.60).
 With SolarBan 70XL argon-filled IGUs: R-value (RSI-value): 4.09 (0.72).
 With 1/4-inch thick multi-wall polycarbonate: R-value (RSI-value): 2.75 (0.48).
 With 3/8-inch thick multi-wall polycarbonate: R-value (RSI-value): 3.21 (0.57).
 With 5/8-inch thick multi-wall polycarbonate: R-value (RSI-value): 3.48 (0.61).
 With 3/8-inch extruded polystyrene-filled solid panels: R-value (RSI-value): 2.60 (0.46).
~~~ END OF MASTER NOTE ~~~~

R-value (RSI-value):  << 2.60 (0.46);  2.75 (0.48);  2.87 (0.51);  2.93 (0.52);  3.17 (0.56);  3.21 (0.57);  3.43 (0.60);  3.48 (0.61); or  4.09 (0.72)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Select operation cycles to coordinate with selected springs.
~~~ END OF MASTER NOTE ~~~~

Operation Cycles:  Capable of operating for minimum << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door Only:  One year.

Panel Material:  Extruded 6063-T6 aluminum.

- ~~~~~ MASTER NOTE ~~~~~~
Models 451, 452, and 453: 1-3/4 inches (45 mm).
Model 464: 1-3/8 inches (35 mm).
Model K-AL: 2 inches (51 mm).

~~~ END OF MASTER NOTE ~~~~

Panel Thickness:  << 1-3/8 inches (35 mm);  1-3/4 inches (45 mm); or  2 inches (51 mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Models 451, 452, and 453 have shiplap joint profile.
Model 464 has tongue and groove joint profile.
Model K-AL has manufacturer's standard with weather seal.
~~~ END OF MASTER NOTE ~~~~

Joint Profile:  << Tongue and groove;  Shiplap; or  Manufacturer's standard with weather seal>>.

Section Height:  Manufacturer's standard based on door height.

- ~~~~~ MASTER NOTE ~~~~~~
Glazing options for models 451, 452, and 453 only.
(DSB - double strength billet)
~~~ END OF MASTER NOTE ~~~~

Glazing:  << Not required;  1/8 inch (3 mm) DSB glass;  1/8 inch (3 mm) tempered DSB glass;  1/4 inch (6 mm) tempered DSB glass;  1/8 inch (3 mm) acrylic;  1/4 inch (6 mm) acrylic;  1/8 inch (3 mm) clear polycarbonate;  1/4 inch (6 mm) clear polycarbonate;  1/4 inch (6 mm) wired glass;  1/2 inch (13 mm) DSB glass;  1/2 inch (13 mm) tempered DSB glass;  1/2 inch (13 mm) acrylic;  1/2 inch (13 mm) polycarbonate;  1/4 inch (6 mm) opaque white;  1/4 inch (6 mm) opaque black;  1/4 inch (6 mm) mirrored gray;  1/4 inch (6 mm) mirrored bronze; or  1/4 inch (6 mm) translucent black>>.

- ~~~~~ MASTER NOTE ~~~~~~
Glazing options for model 464 only.
~~~ END OF MASTER NOTE ~~~~

Glazing:  << Not required;  1/4 inch (6 mm) opaque white;  1/8 inch (3 mm) DSB glass;  1/4 inch (6 mm) opaque black;  1/8 inch (3 mm) tempered DSB glass;  1/4 inch (6 mm) mirrored gray;  1/4 inch (6 mm) tempered DSB glass;  1/4 inch (6 mm) mirrored bronze;  1/8 inch (3 mm) acrylic;  1/4 inch (6 mm) translucent black;  1/4 inch (6 mm) acrylic;  1/8 inch (3 mm) clear polycarbonate;  1/4 inch (6 mm) clear polycarbonate;  1/4 inch (6 mm) wired glass;  1/2 inch (13 mm) DSB glass;  1/2 inch (13 mm) tempered DSB glass;  1/2 inch (13 mm) acrylic; or  1/2 inch (13 mm) polycarbonate>>.

- ~~~~~ MASTER NOTE ~~~~~~
Glazing options for model K-AL only.
(DSB - double strength billet; SSB - single strength billet)
~~~ END OF MASTER NOTE ~~~~

Glazing:  << Not required;  1/8-inch tempered glass, clear;  1/8-inch tempered glass, obscure P-62;  1/8-inch tempered glass, bronze;  1/8-inch tempered glass, satin etch;  1/8-inch polycarbonate, clear;  1/8-inch acrylic, clear;  1/4-inch tempered glass, bronze plate;  1/4-inch tempered glass, clear;  1/4-inch polycarbonate, clear;  7/32-inch laminated glass, white opaque;  1/2-inch IGU, annealed SSB glass, clear;  1/2-inch IGU, annealed SSB glass, Low-E;  1/2-inch IGU, annealed DSB glass, clear;  1/2-inch IGU, tempered DSB glass, Low-E;  1/2-inch IGU, tempered DSB glass, bronze;  1/2-inch IGU, tempered DSB glass, clear;  1/2-inch IGU, tempered DSB glass, gray; or  1/2-inch IGU, tempered DSB glass, satin etch>>.

- ~~~~~ MASTER NOTE ~~~~~~
Finish options per model:
Models 451, 452, and 453: Clear anodized, light bronze anodized, medium bronze anodized, dark bronze anodized, black anodized, powder coat.
Model 464:  Dark bronze anodized, black anodized, powder coated white, powder coated black, powder coated bronze.
Model K-AL: Clear anodized, bronze anodized, black anodized, powder coat.
~~~ END OF MASTER NOTE ~~~~

Exterior Finish:  << Clear anodized;  Light bronze anodized;  Medium bronze anodized;  Dark bronze anodized;  Black anodized; or  Powder coat; color: _____>>.

Locking Options to Include:  << None;  Slide lock; or  Cylinder>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>>.

Weatherstripping and Seals:  << Required; or  Not required>>.

Tracks:  Constructed of steel with members fully bolted together.

Size:  << 2 inches (51 mm); or  3 inches (76 mm)>>.

Finish:  Galvanized steel.

Brackets:  Steel to support counterbalance and curtain.

Finish:  Hot-dip galvanized.

Springs:  Rated for << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up to 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m). Chain hoist over 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m).
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Push-up; or  Chain hoist>>.

Motor Operation:  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction at minimum << 6 inches (152 mm)>>, maximum << 12 inches (305 mm)>> per second.

Timer to Close:  << Not required; or  Automatic closing controlled by adjustable hold-open time delay>>.

Operation Supply Voltage:  << 115/208/230V 1 phase 60Hz;     208/230/460V 3 phase 60Hz;     575V 3 phase 60Hz;     220V 1 phase 50Hz;     220V 3 phase 50Hz; or     400V 3 phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  << Wall mounted; or  Trolley>>, << right-hand side;  left-hand side; or  center>>.

Motor Enclosure:  << Not required;  NEMA 4-12 watertight and oiltight;  NEMA 4X corrosion-resistant;  Totally enclosed, fan-cooled motor; or  Totally enclosed, nonventilated>>.

Obstruction Safety Detection:  Infrared sensor mounted in-plane to door curtain at << 6 inches (152 mm)>> from floor.

Reversing Safety Edge:  << Not required;  Door provided with wireless monitored electric safety edge; or  Door provided with wired monitored electric safety edge>>.

PART 3 EXECUTION

- ~~~~~ MASTER NOTE ~~~~~~
Include requirements for determining suitability of conditions for required work.   Include field testing requirements for materials, substrates, systems, etc.   Specify ambient condition requirements in PART 1 in the FIELD CONDITIONS article.
~~~ END OF MASTER NOTE ~~~~

Examination

Verify wall openings are ready to receive work and opening dimensions and tolerances are within specified limits.

Verify electric power is available and meets manufacturer's requirements.

Preparation

Prepare opening to permit correct installation of door unit to perimeter air and vapor barrier seal.

Apply primer to wood frame.

Installation

Install door unit assembly in accordance with manufacturer's instructions.

Anchor assembly to wall construction and building framing without distortion or stress.

Securely brace door tracks suspended from structure. Secure tracks to structural members only.

Fit and align door assembly, including hardware.

Coordinate installation of electrical service. Complete power and control wiring from disconnect to unit components.

Install << perimeter trim;  closures; and  _____>>.

- ~~~~~ MASTER NOTE ~~~~~~
Limit tolerances to installed dimensions, such as variation in thickness or deviation from intended locations.
~~~ END OF MASTER NOTE ~~~~

Tolerances

Maximum Variation from Plumb:  << 1/16 inch (1.5 mm); or  ___ inch (___ mm)>>.

Maximum Variation from Level:  << 1/16 inch (1.5 mm); or  ___ inch (___ mm)>>.

Longitudinal or Diagonal Warp:  Plus or minus << 1/8 inch in 10 feet (3 mm in 3 m); or  _____ inch in 10 feet (_____ mm in 3 m)>> straight edge.

Maintain dimensional tolerances and alignment with adjacent work.

Adjusting

Adjust door assembly for smooth operation and full contact with weatherstripping.

Confirm operation is in accordance with manufacturer's instructions with present field representative. Identify adjustments to door assembly.

Cleaning

See Section << 017000 - Execution and Closeout Requirements>> for additional requirements.

Clean doors and frames<<  and glazing;    _____; or  None - N/A>>.

Remove<<  temporary;    _____; or  None - N/A>> labels and visible markings.

- ~~~~~ MASTER NOTE ~~~~~~
Describe protection of completed work.
~~~ END OF MASTER NOTE ~~~~

Protection

Protect installed products from damage until Date of Substantial Completion.

Do not permit construction traffic through overhead door openings after adjustment and cleaning.

- END OF SECTION