- ~~~~~ MASTER NOTE ~~~~~~
Steel and aluminum overhead sectional doors; manual and electric; glazing. SECTION 08 3613 - SECTIONAL DOORS - OVERHEAD DOORS (08360 - SECTIONAL OVERHEAD DOORS - OVERHEAD DOORS), Copyright 2025, Wayne Dalton.

- Display hidden notes to specifier. (Don't know how? Click Here)

- ~~~ END OF MASTER NOTE ~~~~

Section 083613 - Sectional Doors – WAYNE DALTON

- PART 1 GENERAL

- Section Includes

Overhead sectional aluminum doors.

~~~~~ MASTER NOTE ~~~~~~
** NOTE TO SPECIFIER **  Delete any sections below not relevant to this project; add others as required.

- ~~~ END OF MASTER NOTE ~~~~

Related Requirements

Section 079200 - Joint Sealants:  Sealing joints between frames and adjacent construction.

Section 087100 - Door Hardware:  Lock cylinders.

- ~~~~~ MASTER NOTE ~~~~~~
This cross-reference is applicable only to doors with electric motor operation.
~~~ END OF MASTER NOTE ~~~~

Section 260533.13 - Conduit for Electrical Systems:  Conduit from electric circuit to operator and from operator to control station, and empty conduit from control units to door operator.

Section 260583 - Wiring Connections.

~~~~~ MASTER NOTE ~~~~~~
** NOTE TO SPECIFIER **  Delete any sections below not relevant to this project; add others as required.

- ~~~ END OF MASTER NOTE ~~~~

Reference Standards

ITS (DIR) - Directory of Listed Products; Current Edition.

UL (DIR) - Online Certifications Directory; Current Edition.

UL 325 - Standard for Door, Drapery, Gate, Louver, and Window Operators and Systems; Current Edition, Including All Revisions.

- ~~~~~ MASTER NOTE ~~~~~~
Do not request submittals if drawings sufficiently describe the products of this section or if proprietary specifying techniques are used. The review of submittals increases the possibility of unintended variations from contract documents, thereby increasing the design professional's liability.   See Section 01 3000 - Administrative Requirements for definitions of the types of submittals for review, information, and project record.
~~~ END OF MASTER NOTE ~~~~

Submittals

See Section 013000 - Administrative Requirements for submittal procedures.

Product Data:  Submit manufacturer's standard literature showing materials and details of construction and finish.<<  Include data on electrical operation.; or  None - N/A>>

Shop Drawings:  Indicate rough and actual opening dimensions, anchorage methods, hardware locations, and installation details.

- ~~~~~ MASTER NOTE ~~~~~~
Include this paragraph for submission of physical samples to verify selected finish, color, texture, etc.
~~~ END OF MASTER NOTE ~~~~

Samples:  << Two; or  _____>> panel samples or color chips, << 4 inches long (102 mm long); or  ___ inches long (___ mm long)>>, illustrating shape, color, and finish texture.

- ~~~~~ MASTER NOTE ~~~~~~
When manufacturer's instructions for specific installation requirements are referenced in PART 3, include this request for submittal of those instructions. Edit PART 3 statements to avoid conflict with manufacturer's instructions.
~~~ END OF MASTER NOTE ~~~~

Manufacturer's Installation Instructions:  Indicate installation sequence and installation, adjustment, and alignment procedures.

- ~~~~~ MASTER NOTE ~~~~~~
Request qualification statements if qualifications are defined in PART 1 under QUALITY ASSURANCE article.
~~~ END OF MASTER NOTE ~~~~

Manufacturer's qualification statement.

- ~~~~~ MASTER NOTE ~~~~~~
Request qualification statements if qualifications are defined in PART 1 under QUALITY ASSURANCE article.
~~~ END OF MASTER NOTE ~~~~

Installer's qualification statement.

Operation and Maintenance Data:  Indicate modes of operation, lubrication requirements and frequency, and periodic adjustments required.

Specimen warranty.

- ~~~~~ MASTER NOTE ~~~~~~
Coordinate this paragraph with WARRANTY article.
~~~ END OF MASTER NOTE ~~~~

Executed warranty.

Project Record Documents:  Include as-built electrical diagrams for electrical operation and connection to fire alarm system.

Quality Assurance

Manufacturer Qualifications:  Company specializing in manufacturing products specified in this section with minimum << 5 years; or  _______ years>> of<<  documented; or  None - N/A>> experience.

Installer Qualifications:  Company specializing in performing work of type specified and with minimum << 3 years; or  _____ years>> of<<  documented;    _____; or  None - N/A>> experience<< and approved by manufacturer; or  None - N/A>>.

Comply with << applicable; or  ________>> code for motor and motor control requirements.

- ~~~~~ MASTER NOTE ~~~~~~
Full citation found in the REFERENCE STANDARDS article. Click on the link under the REFERENCE STANDARDS article for free information or to purchase a copy of this document.
~~~ END OF MASTER NOTE ~~~~

Products Requiring Electrical Connection:  Listed and classified by << ITS (DIR);  UL (DIR);  testing firm acceptable to authorities having jurisdiction; or  _____>>.

- ~~~~~ MASTER NOTE ~~~~~~
Improper specifying of warranty and correction period extensions may limit the Owner's statutory rights and the ability to enforce claims under the Contractor's contractual warranty. Be careful about establishing warranty requirements that few, if any, manufacturers will honor.
~~~ END OF MASTER NOTE ~~~~

Warranty

See Section 017800 - Closeout Submittals for additional warranty requirements.

- ~~~~~ MASTER NOTE ~~~~~~
This paragraph requests a manufacturer warranty; the request may not be effective as the manufacturer is outside the jurisdiction of the Owner/Contractor contract. Coordinate this paragraph with the SUBMITTALS article.
~~~ END OF MASTER NOTE ~~~~

Manufacturer Warranty:  Provide manufacturer warranty for counterbalance shaft assembly for years indicated under individual doors. Complete forms in Owner's name and register with manufacturer.

PART 2 PRODUCTS

Manufacturers

- ~~~~~ MASTER NOTE ~~~~~~
This section is based on the products of Wayne Dalton, which is located at:

- 2501 S. State Highway 121 Business, Suite 200

- Lewisville, TX 75067

- Phone: (800) 827-3667

- Web Site: www.wayne-dalton.com

- Email: info@wayne-dalton.com.

- [click Here] for additional information.

- Since its inception in 1954, Wayne Dalton has become known as a company with innovative ideas, which far exceed industry standards. Often, Wayne Dalton is the only source for the latest garage door and garage door opener features. Because the company has always maintained a staunch commitment to developing innovative new products, Wayne Dalton is now a world leader in the garage door and garage door opener industry.

- Wayne Dalton Rolling Doors have a long history of excellence in the design and construction of doors that have met and often exceeded the needs and expectations of even the most critical projects.

- With numerous innovations created and experience acquired over the years, Wayne Dalton continues to lead all other manufacturers with both standard and custom-made doors from a variety of materials and colors to meet almost any need.

- So whether it's enormous Titan rolling doors, protective FireStar rolling steel fire doors, ventilated Secur-Vent doors, or secure Accordion-Folding Grilles, you can feel confident that with Wayne Dalton's many years of knowledge and experience, you will get the best possible solution for your building application needs.

- ~~~ END OF MASTER NOTE ~~~~

Wayne Dalton<< ; _______; or  None - N/A>>:  https://www.wayne-dalton.com/

__________.

__________.

Substitutions:  << See Section 016000 - Product Requirements; or  Not permitted>>.

Overhead Sectional Aluminum Doors

- ~~~~~ MASTER NOTE ~~~~~~
Insert drawing designation here, if applicable.
~~~ END OF MASTER NOTE ~~~~

<< None - N/A; or  _______ : >>Aluminum Sectional Doors:  Wayne Dalton; <<Model 453>>.

Panels:  Aluminum construction and reinforcement.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum widths:
Model 453: 22 feet 2 inches (6.7 m)

For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:   _____ feet _____ inches (_____ m).

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum heights: 
Model 453: 20 feet 1 inch (6.1 m).

For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:   _____ feet _____ inch (_____ m).

Mounting:  Surface mounted on interior of building.

Opening Speed:  Door to operate at variable speed of 6 to 12 inches (152 to 305 mm) per second.

Closing Speed:  Door to operate at variable speed of 6 to 12 inches (152 to 305 mm) per second.

- ~~~~~ MASTER NOTE ~~~~~~
U-factor values:
Model 453 with double strength billet (DSB) clear, tempered, or obscure glazing: 0.30.
Model 453 with DSB Low-E coated glazing: 0.28.
~~~ END OF MASTER NOTE ~~~~

U-Factor:  << Not required;  0.30; or  0.28>>.

- ~~~~~ MASTER NOTE ~~~~~~
R-value (RSI-value) for Model 453:
With double strength billet (DSB) clear, tempered, or obscure glazing IGUs: R-value (RSI-value): 2.87 (0.51).
With clear polycarbonate IGUs: R-value (RSI-value): 2.93 (0.52).
With solar bronze IGUs: R-value (RSI-value): 3.17 (0.56).
With Low-E coating IGUs: R-value (RSI-value): 3.43 (0.60).
With SolarBan 70XL argon-filled IGUs: R-value (RSI-value): 4.09 (0.72).
With 1/4-inch thick multi-wall polycarbonate: R-value (RSI-value): 2.75 (0.48).
With 3/8-inch thick multi-wall polycarbonate: R-value (RSI-value): 3.21 (0.57).
With 5/8-inch thick multi-wall polycarbonate: R-value (RSI-value): 3.48 (0.61).
With 3/8-inch extruded polystyrene-filled solid panels: R-value (RSI-value): 2.60 (0.46).
~~~ END OF MASTER NOTE ~~~~

R-value (RSI-value):  << 2.60 (0.46);  2.75 (0.48);  2.87 (0.51);  2.93 (0.52);  3.17 (0.56);  3.21 (0.57);  3.43 (0.60);  3.48 (0.61); or  4.09 (0.72)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Select operation cycles to coordinate with selected springs.
~~~ END OF MASTER NOTE ~~~~

Operation Cycles:  Capable of operating for minimum << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door Only:  One year.

Panel Material:  Extruded 6063-T6 aluminum.

Panel Thickness:     1-3/4 inches (45 mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Model 453 has a shiplap joint profile.

~~~ END OF MASTER NOTE ~~~~

Joint Profile:  <<Shiplap>>.

Section Height:  Manufacturer's standard based on door height.

- ~~~~~ MASTER NOTE ~~~~~~
Glazing options available per model:
Models 453: 1/8 inch (3 mm) DSB glass, 1/8 inch (3 mm) and 1/4 inch (6 mm) tempered DSB glass, 1/8 inch (3mm) and 1/4 inch (6 mm) acrylic, 1/8 inch (3 mm) and 1/4 inch (6 mm) clear polycarbonate, 1/4 inch (6 mm) wired glass.
Insulated DSB vision lites: 1/2 inch (13 mm) DSB glass, 1/2 inch (13 mm) tempered DSB glass, 1/2 inch (13 mm) acrylic, 1/2 inch (13 mm) polycarbonate.

~~~ END OF MASTER NOTE ~~~~

Glazing:  << Not required;  1/8 inch (3 mm) DSB glass;  1/8 inch (3 mm) tempered DSB glass;  1/4 inch (6 mm) tempered DSB glass;  1/8 inch (3 mm) acrylic;  1/4 inch (6 mm) acrylic;  1/8 inch (3 mm) clear polycarbonate;  1/4 inch (6 mm) clear polycarbonate;  1/4 inch (6 mm) wired glass;  1/2 inch (13 mm) DSB glass;  1/2 inch (13 mm) tempered DSB glass;  1/2 inch (13 mm) acrylic;  1/2 inch (13 mm) polycarbonate;  1/4 inch (6 mm) opaque white;  1/4 inch (6 mm) opaque black;  1/4 inch (6 mm) mirrored gray;  1/4 inch (6 mm) mirrored bronze; or  1/4 inch (6 mm) translucent black>>.

- ~~~~~ MASTER NOTE ~~~~~~
Finish options per model:
Models 453: Clear anodized, light bronze anodized, medium bronze anodized, dark bronze anodized, black anodized, powder coat.

~~~ END OF MASTER NOTE ~~~~

Exterior Finish:  << Clear anodized;  Light bronze anodized;  Medium bronze anodized;  Dark bronze anodized;  Black anodized; or  Powder coat; color: _____>>.

Locking Options to Include:  << None;  Slide lock; or  Cylinder>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>>.

Weatherstripping and Seals:  << Required; or  Not required>>.

Tracks:  Constructed of steel with members fully bolted together.

Size:  << 2 inches (51 mm); or  3 inches (76 mm)>>.

Finish:  Galvanized steel.

Brackets:  Steel to support counterbalance and curtain.

Finish:  Hot-dip galvanized.

Springs:  Rated for << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up to 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m). Chain hoist over 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m).
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Push-up; or  Chain hoist>>.

Motor Operation:  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction at minimum 6 inches (152 mm), maximum 12 inches (305 mm) per second.

Timer to Close:  << Not required; or  Automatic closing controlled by adjustable hold-open time delay>>.

Operation Supply Voltage:  << 115/208/230V 1 phase 60Hz;    208/230/460V 3 phase 60Hz;    575V 3 phase 60Hz;    220V 1 phase 50Hz;    220V 3 phase 50Hz; or    400V 3 phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  << Wall mounted; or  Trolley>>, << right-hand side;  left-hand side; or  center>>.

Motor Enclosure:  << Not required;  NEMA 4-12 watertight and oiltight;  NEMA 4X corrosion-resistant;  Totally enclosed, fan-cooled motor; or  Totally enclosed, non-ventilated>>.

Obstruction Safety Detection:  Infrared sensor mounted in-plane to door curtain at 6 inches (152 mm) from floor.

Reversing Safety Edge:  << Not required;  Door provided with wireless monitored electric safety edge; or  Door provided with wired monitored electric safety edge>>.

- ~~~~~ MASTER NOTE ~~~~~~
Insert drawing designation here, if applicable.
~~~ END OF MASTER NOTE ~~~~

PART 3 EXECUTION

Examination

Verify wall openings are ready to receive work and opening dimensions and tolerances are within specified limits.

Verify electric power is available and meets manufacturer's requirements.

Preparation

Prepare opening to permit correct installation of door unit to perimeter air and vapor barrier seal.

Apply primer to wood frame.

Installation

Install door unit assembly in accordance with manufacturer's instructions.

Anchor assembly to wall construction and building framing without distortion or stress.

Securely brace door tracks suspended from structure. Secure tracks to structural members only.

Fit and align door assembly including hardware.

Coordinate installation of electrical service. Complete power and control wiring from disconnect to unit components.

Install << perimeter trim;  closures; and  ___________>>.

Tolerances

Maximum Variation from Plumb:  << 1/16 inch (1.5 mm); or  ___ inch (___ mm)>>.

Maximum Variation from Level:  << 1/16 inch (1.5 mm); or  ___ inch (___ mm)>>.

Longitudinal or Diagonal Warp:  Plus or minus << 1/8 inch in 10 feet (3 mm in 3 m); or  _____ inch in 10 feet (_____ mm in 3 m)>> straight edge.

Maintain dimensional tolerances and alignment with adjacent work.

Adjusting

Adjust door assembly for smooth operation and full contact with weatherstripping.

Confirm operation is in accordance with manufacturer's instructions with present field representative. Identify adjustments to door assembly.

Cleaning

See Section 017000 - Execution and Closeout Requirements for additional requirements.

Clean doors and frames<<  and glazing;    _____; or  None - N/A>>.

Remove<<  temporary;    _____; or  None - N/A>> labels and visible markings.

Protection

Protect installed products from damage until Date of Substantial Completion.

Do not permit construction traffic through overhead door openings after adjustment and cleaning.

END OF SECTION  083613