Section 083313
coiling COUNTER DOORS
Model 500

NOTE: Enable Microsoft Word option to display hidden text to view notes to specifier.

PART 1 GENERAL

Section Includes

Metal coiling counter doors.

Related Requirements

Section << 061000 - Rough Carpentry>>:  Rough openings.

Section << 079200 - Joint Sealants>>:  Sealing joints between frames and adjacent construction.

Section << 087100 - Door Hardware>>:  Cylinder cores and keys.

Section << 260583 - Wiring Connections>>:  Power to disconnect.

Reference Standards

ASTM A240/A240M - Standard Specification for Chromium and Chromium-Nickel Stainless Steel Plate, Sheet, and Strip for Pressure Vessels and for General Applications; 2025a.

ITS (DIR) - Directory of Listed Products; Current Edition.

UL (DIR) - Online Certifications Directory; Current Edition.

~~~~~ MASTER NOTE ~~~~~~
Do not request submittals if drawings sufficiently describe the products of this section or if proprietary specifying techniques are used. The review of submittals increases the possibility of unintended variations from contract documents, thereby increasing the design professional's liability.

- ~~~ END OF MASTER NOTE ~~~~

Submittals

Product Data:  Provide details of general construction component connections<< and electrical equipment;   _____; or  None - N/A>>.

Shop Drawings:  Indicate dimensioning, anchorage methods, hardware locations, and installation details.

Manufacturer's Instructions:  Indicate installation sequences and procedures, as well as adjustment and alignment procedures.

Manufacturer's qualification statement.

Installer's qualification statement.

Operation and Maintenance Data:  Indicate << lubrication requirements and frequency;  periodic adjustments required; and  _____>>.

Specimen warranty.

Quality Assurance

Manufacturer Qualifications:  Company specializing in manufacturing products specified in this section with minimum << 3 years; or  _____>> of<<  documented;    _____; or  None - N/A>> experience.

Installer Qualifications:  Company specializing in performing work of type specified and with minimum << 3 years; or  _____>> of<<   documented;    _____; or  None - N/A>> experience<< and approved by manufacturer; or  None - N/A>>.

Products Requiring Electrical Connection:  Listed and classified by << ITS (DIR);  UL (DIR);  testing firm acceptable to authorities having jurisdiction; or  _____>> suitable for purpose specified and indicated.

Delivery, Storage, and Handling

Store products in manufacturer's unopened packaging until ready for installation.

Protect materials from exposure to moisture.

Store materials in dry, warm, ventilated, weathertight location.

Warranty

Manufacturer Warranty:

Provide manufacturer warranty for counterbalance shaft assembly for years indicated under individual doors.

Include warranty for product and components against defects in materials and workmanship for << two; or  _____>> years, measured from date of installation.

PART 2 PRODUCTS

Manufacturer

- ~~~~~ MASTER NOTE ~~~~~~
Founded in 1954, Wayne Dalton was built on a dedication to ingenuity and customer service. As our business grew, so did our commitment to expanding our product lines, leading the company to become one of the largest door manufacturers in the United States. Since the beginning, we have remained committed to being the garage door partner whose innovation drives dealer productivity and provides industry-leading garage door solutions. In continuing this innovation, Wayne Dalton offers products, not only through our dealer connections, but also through our retail partner, Lowe's. Wayne Dalton offers a broad selection of residential garage doors, as well as a complete line of doors for commercial and industrial applications. Our Commercial Sectional Doors are ideal for jobs that call for excellent thermal efficiency, have stringent wind load requirements, or simply need a high performing door designed for long life and superior strength. Our Commercial Rolling Doors include service doors, counter doors, fire doors, security grilles, security shutters and roll up sheet doors. In 2016, we expanded our commercial door category to include High Speed Doors. Wayne Dalton designed and built a new series of High-Speed Doors from the ground up. These doors feature patent-pending, high-cycle designs, and industry-leading safety features for both interior and exterior applications. With everything from residential garage doors to rolling doors to commercial sectional to high performance doors, Wayne Dalton’s breadth of product can provide a garage door solution for any application.   Contact Information: Wayne Dalton 2501 S State Hwy 121 Bus, Suite 200 Lewisville, Texas 75067 United States   Phone: (800) 827-3667 Email: info@wayne-dalton.com   Contact for Design Professional: Customer Service Phone: (800) 827-3667 Email: info@wayne-dalton.com   Online CEU Courses   Request Lunch & Learn   https://www.wayne-dalton.com/   Plant Location: Dalton, Ohio; Mt. Hope, Ohio, Pensacola, Florida; Centralia, Washington.    
~~~ END OF MASTER NOTE ~~~~

Wayne Dalton; www.wayne-dalton.com; 1 (800) 827-3667.

Substitutions: Not permitted.

METAL COILING COUNTER DOORS

Wayne Dalton, Model 500.

Slats:  Interlocking, roll-formed, metal slats.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 18 feet (5486 mm) for aluminum. Standard maximum width of 15 feet (4572 mm) for steel and stainless steel. For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet (_____ mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 7 feet (2133 mm). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet (_____ mm)>>.

Mounting:  << Surface-mounted on side indicated on drawings;  Between jamb; or  _____>>.

Opening Speed:  Door to operate at variable speed up to << 8 inches (203 mm)>> per second.

Closing Speed:  Door to operate at variable speed up to << 12 inches (305 mm)>> per second.

Door Warranty:  Two years.

Curtain Material:  << 22-gauge, 0.0336-inch (0.853 mm) galvanized steel;  22-gauge, 0.0336-inch (0.853 mm) stainless steel;  22-gauge, 0.0336-inch (0.853 mm) G-90 galvanized steel; or  16-gauge, 0.05-inch (0.853 mm) aluminum>>; endlocks attached to alternate slats maintain curtain alignment and prevent lateral slat movement.

Slat Profile:  No.17 << 2-inch (51 mm)>> flat slats. Fit alternate slats with end locks to maintain curtain alignment.

- ~~~~~ MASTER NOTE ~~~~~~
1/16-inch (1.6 mm) diameter hole offering 20 to 25 percent open area over length of each slat. Only available in steel and stainless steel
~~~ END OF MASTER NOTE ~~~~

Perforated Slats:  Secur-Vent perforated holes.

Finish:

- ~~~~~ MASTER NOTE ~~~~~~
Standard color for guides and bottom bar is matching curtain. Approximately 200 optional RAL powder-coat colors are also available.
G-90 galvanized only available in G-90 Gray.
~~~ END OF MASTER NOTE ~~~~

Standard polyester base coat. Color:  << Gray;  Brown;  Beige;  White;  G-90 Gray; or  _____>>.

- ~~~~~ MASTER NOTE ~~~~~~
Polyester powder coat available in approximately 200 optional RAL powder coat colors; custom color match options available.
~~~ END OF MASTER NOTE ~~~~

<< Powder coat; or  _____>>.<<  Color: _____ .; or  None - N/A>>

Aluminum Finish:  << Clear, anodized;  Bronze, anodized; or  _____>>.

Stainless Steel Finish:  No.4 finish.

Bottom Bar:  << Aluminum;  Galvanized steel;  Stainless steel; or  _____>>.

Profile:  << Extruded aluminum;  Tubular;  Single angle; or  _____>>.

- ~~~~~ MASTER NOTE ~~~~~~
Polyester powder coat available in approximately 200 optional RAL powder coat colors; custom color match options available.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Black anodized;  Black powder coat;  Bronze anodized;  Clear anodized;  Galvanized;  Match curtain;  Powder coat; or  _____>>.<< Color:  _____ .; or  None - N/A>>

Locking Options to Include:  << Best cylinder.;  Cylinder.;  Slide lock.;  _____ .; or  None - N/A>>

Lock Locations:  << Both jambs;  Left jamb only;  Right jamb only; or  _____>>.

Lock Access:  Coil side.

Weatherstripping and Seals:

Bottom Seal Astragal:  << Required;  Not required; or  _____>>.

Side Guides, Channels:  Constructed of << extruded aluminum;  steel;  304 stainless steel; or  _____>> with members fully bolted together. Include continuous strips of wool pile in guides to eliminate metal-to-metal contact and to provide dust-seal around curtain.

Finish:  << Black anodized;  Black powder coat;  Bronze anodized;  Clear anodized;  Match curtain;  No.4 satin;  Powder coat; or  _____>>.<< Color:  _____ .; or  None - N/A>>

Brackets:  << 0.13-inch (3.2 mm) extruded aluminum;  0.18-inch (4.8 mm) galvanized steel; or  0.18-inch (4.8 mm) stainless steel>> to support counterbalance assembly and curtain.

Finish:  << Match curtain;  Powder coat; or  _____>>.<< Color:  _____ .; or  None - N/A>>

Counterbalance Assembly:  Helical-torsion-spring type, housed in steel tube or pipe barrel and supporting curtain with deflection limited to << 0.03 inch per foot (2.5 mm per m)>> of span; adjustable spring tension required.

Hood:  Encloses curtain coil and counterbalance mechanism; fabricated of sheet metal with flanges for attachment and longitudinal stiffness.

Manufacturer's standard sheet metal.

<< 24-gauge, 0.025-inch (0.635 mm)>> stainless steel.

<< 24-gauge, 0.0276-inch (0.701 mm)>> galvanized steel.

<< 22-gauge, 0.025-inch (0.64 mm)>> aluminum.

Finish:

- ~~~~~ MASTER NOTE ~~~~~~
Standard color for guides and bottom bar is matching curtain. Approximately 180 optional RAL powder-coat colors are also available.
G-90 galvanized only available in G-90 Gray.
~~~ END OF MASTER NOTE ~~~~

Standard polyester base coat. Color:  << Gray;  Brown;  Beige;  White;  G-90 Gray; or  _____>>.

- ~~~~~ MASTER NOTE ~~~~~~
Polyester powder coat available in 197 colors; custom color match options available.
Textured powder coat available in 11 colors.
~~~ END OF MASTER NOTE ~~~~

<< Powder coat;  Textured powder coat; or  _____>>.<<  Color: _____ .; or  None - N/A>>

Aluminum Finish:  << Clear, anodized;  Bronze, anodized; or  _____>>.

Stainless Steel Finish:  No.4 finish.

Manual Operation:  << Push-up; or  Hand crank>>.

- ~~~~~ MASTER NOTE ~~~~~~
Tube motor only available with door wider than 2 feet, 6 inches (762 mm) and less than 14 feet (4267 mm), where height is no more than 6 feet (1829 mm).
~~~ END OF MASTER NOTE ~~~~

Motor Operation:  << Standard motor;  Tube-mounted motor; or  _____>>.

Override:  << Crank;  Manual; or  _____>>.

Tube Motor Configuration:

Timer to Close:  Not applicable.

Operation Supply Voltage:  115/208/230 V, 1-phase, 60 Hz.

Signaling Device:  Not applicable.

Actuation:  Provide push button.

Motor Mounting:  Within tube.

Obstruction Safety Detection:  Not applicable.

Standard Motor Configuration:

Timer to Close:  << Automatic closing controlled by adjustable hold-open time delay;  Not required; or  _____>>.

Operation Supply Voltage:  115/208/230 V, 1-phase, 60 Hz.

Signaling Device:  << Horn and strobe combination;  Traffic warning light; or  _____>>.

Actuation:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector; and  _____>>.

Motor Mounting:  << Wall-mounted;  Front of hood; or  _____>>, << right-hand side;  left-hand side; or  _____>>.

Obstruction Safety Detection:

Constant Contact Button:  << Required;  Not required; or  _____>>.

Photo Eyes:  << NEMA 4X;  Protected;  Reflective;  Standard;  None; or  _____>>.

Light Curtain:  << 3 feet (0.9 m);  6 feet (1.8 m); or  _____ feet (_____ m)>>.

Reversing Safety Edge:  << Door provided with wireless, monitored electric safety edge;  Door provided with wired, monitored safety edge;  Not required; or  _____>>.

PART 3  EXECUTION

- ~~~~~ MASTER NOTE ~~~~~~
Include requirements for determining suitability of conditions for required work.
Include field testing requirements for materials, substrates, systems, etc.
Specify ambient condition requirements in PART 1 in the FIELD CONDITIONS article.
~~~ END OF MASTER NOTE ~~~~

Examination

Verify adjacent construction is suitable for door installation.

Verify electrical services have been installed and are accessible.

Verify door opening is plumb, header is level, and dimensions are correct.

Notify <<Architect>> of unacceptable conditions or varying dimensions.

Commencement of installation indicates acceptance of substrate and door opening conditions.

Installation

Install units in accordance with manufacturer's instructions.

Use anchorage devices to securely fasten assembly to wall construction and building framing without distortion or stress.

Securely and rigidly brace components suspended from structure.<<  Secure guides to structural members only.;    _____ .; or  None - N/A>>

Fit and align assembly including hardware; level and plumb to provide smooth operation.

Coordinate installation of electrical service; see Section << 260583>>.

Complete wiring from disconnect to unit components.

Complete wiring from fire alarm system.

Install perimeter trim as shown on drawings.

- ~~~~~ MASTER NOTE ~~~~~~
Limit tolerances to installed dimensions, such as variation in thickness or deviation from intended locations.
~~~ END OF MASTER NOTE ~~~~

Tolerances

Maintain dimensional tolerances and alignment with adjacent work.

Maximum Variation from Plumb:  << 1/16 inch (1.5 mm); or  _____ inch (_____ mm)>>.

Maximum Variation from Level:  << 1/16 inch (1.5 mm); or  _____ inch (_____ mm)>>.

Longitudinal or Diagonal Warp:  Plus or minus << 1/8 inch per 10 feet (3 mm per 3 m); or  _____ inch per 10 feet (_____ mm per 3 m)>> straight edge.

Adjusting

Adjust operating assemblies for smooth and noiseless operation.

Cleaning

See Section << 017000 - Execution and Closeout Requirements>> for additional requirements.

Clean installed components.

Remove labels and visible markings.

- END OF SECTION