Section 083313
coiling COUNTER DOOR
model 560

NOTE: Enable Microsoft Word option to display hidden text to view notes to specifier.

PART 1 GENERAL

Section Includes

Metal coiling counter doors with integral frame and sill.

Related Requirements

Section << 061000 - Rough Carpentry>>:  Rough openings.

Section << 079200 - Joint Sealants>>:  Sealing joints between frames and adjacent construction.

Section << 087100 - Door Hardware>>:  Cylinder cores and keys.

Section << 260583 - Wiring Connections>>:  Power to disconnect.

Reference Standards

ASTM A240/A240M - Standard Specification for Chromium and Chromium-Nickel Stainless Steel Plate, Sheet, and Strip for Pressure Vessels and for General Applications; 2025a.

ITS (DIR) - Directory of Listed Products; Current Edition.

UL (DIR) - Online Certifications Directory; Current Edition.

~~~~~ MASTER NOTE ~~~~~~
Do not request submittals if drawings sufficiently describe the products of this section or if proprietary specifying techniques are used. The review of submittals increases the possibility of unintended variations from contract documents, thereby increasing the design professional's liability.

- ~~~ END OF MASTER NOTE ~~~~

Submittals

Product Data:  Provide details of general construction component connections<< and electrical equipment;   _____; or  None - N/A>>.

Shop Drawings:  Indicate dimensioning, anchorage methods, hardware locations, and installation details.

Manufacturer's Instructions:  Indicate installation sequences and procedures, as well as adjustment and alignment procedures.

Manufacturer's qualification statement.

Installer's qualification statement.

Operation and Maintenance Data:  Indicate << lubrication requirements and frequency;  periodic adjustments required; and  _____>>.

Specimen warranty.

Quality Assurance

Manufacturer Qualifications:  Company specializing in manufacturing products specified in this section with minimum << 3 years; or  _____>> of<<  documented;    _____; or  None - N/A>> experience.

Installer Qualifications:  Company specializing in performing work of type specified and with minimum << 3 years; or  _____>> of<<   documented;    _____; or  None - N/A>> experience<< and approved by manufacturer; or  None - N/A>>.

Products Requiring Electrical Connection:  Listed and classified by << ITS (DIR);  UL (DIR);  testing firm acceptable to authorities having jurisdiction; or  _____>> suitable for purpose specified and indicated.

Delivery, Storage, and Handling

Store products in manufacturer's unopened packaging until ready for installation.

Protect materials from exposure to moisture.

Store materials in dry, warm, ventilated, weathertight location.

Warranty

Manufacturer Warranty:

Provide manufacturer warranty for counterbalance shaft assembly for years indicated under individual doors.

Include warranty for product and components against defects in materials and workmanship for << two; or  _____>> years, measured from date of installation.

Complete forms in <<Owner>>'s name and register with manufacturer.

PART 2 PRODUCTS

Manufacturer

- ~~~~~ MASTER NOTE ~~~~~~
Founded in 1954, Wayne Dalton was built on a dedication to ingenuity and customer service. As our business grew, so did our commitment to expanding our product lines, leading the company to become one of the largest door manufacturers in the United States. Since the beginning, we have remained committed to being the garage door partner whose innovation drives dealer productivity and provides industry-leading garage door solutions. In continuing this innovation, Wayne Dalton offers products, not only through our dealer connections, but also through our retail partner, Lowe's. Wayne Dalton offers a broad selection of residential garage doors, as well as a complete line of doors for commercial and industrial applications. Our Commercial Sectional Doors are ideal for jobs that call for excellent thermal efficiency, have stringent wind load requirements, or simply need a high performing door designed for long life and superior strength. Our Commercial Rolling Doors include service doors, counter doors, fire doors, security grilles, security shutters and roll up sheet doors. In 2016, we expanded our commercial door category to include High Speed Doors. Wayne Dalton designed and built a new series of High-Speed Doors from the ground up. These doors feature patent-pending, high-cycle designs, and industry-leading safety features for both interior and exterior applications. With everything from residential garage doors to rolling doors to commercial sectional to high performance doors, Wayne Dalton’s breadth of product can provide a garage door solution for any application.   Contact Information: Wayne Dalton 2501 S State Hwy 121 Bus, Suite 200 Lewisville, Texas 75067 United States   Phone: (800) 827-3667 Email: info@wayne-dalton.com   Contact for Design Professional: Customer Service Phone: (800) 827-3667 Email: info@wayne-dalton.com   Online CEU Courses   Request Lunch & Learn   https://www.wayne-dalton.com/   Plant Location: Dalton, Ohio; Mt. Hope, Ohio, Pensacola, Florida; Centralia, Washington.    
~~~ END OF MASTER NOTE ~~~~

Wayne Dalton; www.wayne-dalton.com; 1 (800) 827-3667.

Substitutions: Not permitted.

Metal Coiling Counter Doors with Integral Frame and Sill

Wayne Dalton, Model 560.

Slats:  Interlocking, roll-formed, metal slats.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 18 feet (5486 mm) for aluminum. Standard maximum width of 15 feet (4572 mm) for steel and stainless steel. 
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet (_____ mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 6 feet (1828 mm). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet (_____ mm)>>.

Mounting:  Between jamb.

Door Warranty:  One year.

Curtain Material:  << 0.040-inch (1.016 mm)>> aluminum.

Slat Profile:  << 1-1/4 inches (32 mm)>>, flat.

Endlocks:  Hot-dip galvanized, stamped components riveted to each end of alternate slats to prevent lateral movement and limit both slat deflection and bending stress.

- ~~~~~ MASTER NOTE ~~~~~~
Polyester powder coat available in approximately 200 optional RAL powder coat colors; custom color match options available.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Black anodized;  Bronze anodized;  Clear anodized;  Powder coat; or  _____>>.<<  Color: _____ .; or  None - N/A>>

Bottom Bar:  6063-T6 aluminum.

Finish:  Clear anodized.

Locking Options to Include:  << Best cylinder;  Cylinder;  Slide lock; or  _____>>.

Lock Locations:  << Both jambs;  Left jamb only;  Right jamb only; or  _____>>.

Lock Access:  << Both sides;  Coil side;  Opposite coil side; or  _____>>.

Weatherstripping and Seals:

Bottom Seal Astragal:  << Required;  Not required; or  _____>>.

Guide Seal:  << Brush, one-sided;  Not required; or  _____>>.

Brackets:  Minimum << 12-gauge, 0.1046-inch (2.66 mm)>> steel plate to support counterbalance assembly and form end closures. Provide ball bearings at rotating support points. Bolt plates to mounting structures with minimum << 1/4-inch (6 mm)>> fasteners.

Material:  <<ASTM A240/A240M>>, stainless steel.

Finish:  No.2B mill finish.

Counterbalance Assembly:  Steel pipe barrel capable of carrying curtain load with maximum deflection of << 0.03-inch per foot (2.5 mm per m)>> of door width.

Material:  Heat-treated, helical-torsion springs, encased in steel pipe and designed to include overload factor of 25 percent to ensure minimum effort to operate.

Assembly:  Sealed and prelubricated high-speed ball bearing at rotating support points. Torsion spring charge wheel for applying spring torque and for future adjustments.

Hood:  <<ASTM A240/A240M>>, << 16-gauge, 0.625-inch (1.587 mm)>> stainless steel, formed to fit contour of end brackets with reinforced top and bottom edges.

Shape:  Square.

Finish:  No.4 satin.

Integral Frame:  <<ASTM A240/A240M>>, << 16-gauge, 0.625-inch (1.587 mm)>> stainless steel, with guides incorporated into jamb design, reinforced by << 12-gauge, 0.1046-inch (2.66 mm)>> mounting structure.

Finish:  No.4 satin.

Sill:

Manufacturer's standard sheet metal, << 14-gauge, 0.078-inch (1.98 mm)>> stainless steel.

Finish:  No.4 satin.

Not required.

Manual Operation:  << Hand crank;  Push-up; or  _____>>.

PART 3  EXECUTION

- ~~~~~ MASTER NOTE ~~~~~~
Include requirements for determining suitability of conditions for required work.
Include field testing requirements for materials, substrates, systems, etc.
Specify ambient condition requirements in PART 1 in the FIELD CONDITIONS article.
~~~ END OF MASTER NOTE ~~~~

Examination

Verify adjacent construction is suitable for door installation.

Verify electrical services have been installed and are accessible.

Verify door opening is plumb, header is level, and dimensions are correct.

Notify <<Architect>> of unacceptable conditions or varying dimensions.

Commencement of installation indicates acceptance of substrate and door opening conditions.

Installation

Install units in accordance with manufacturer's instructions.

Use anchorage devices to securely fasten assembly to wall construction and building framing without distortion or stress.

Securely and rigidly brace components suspended from structure.<<  Secure guides to structural members only.;    _____ .; or  None - N/A>>

Fit and align assembly including hardware; level and plumb to provide smooth operation.

Coordinate installation of electrical service; see Section << 260583>>.

Complete wiring from disconnect to unit components.

Complete wiring from fire alarm system.

Install perimeter trim as shown on drawings.

- ~~~~~ MASTER NOTE ~~~~~~
Limit tolerances to installed dimensions, such as variation in thickness or deviation from intended locations.
~~~ END OF MASTER NOTE ~~~~

Tolerances

Maintain dimensional tolerances and alignment with adjacent work.

Maximum Variation from Plumb:  << 1/16 inch (1.5 mm); or  _____ inch (_____ mm)>>.

Maximum Variation from Level:  << 1/16 inch (1.5 mm); or  _____ inch (_____ mm)>>.

Longitudinal or Diagonal Warp:  Plus or minus << 1/8 inch per 10 feet (3 mm per 3 m); or  _____ inch per 10 feet (_____ mm per 3 m)>> straight edge.

Adjusting

Adjust operating assemblies for smooth and noiseless operation.

Cleaning

See Section << 017000 - Execution and Closeout Requirements>> for additional requirements.

Clean installed components.

Remove labels and visible markings.

- END OF SECTION