Section 083323.13
rapid upward coiling Doorss

models 885, 885LH

NOTE: Enable Microsoft Word option to display hidden text to view notes to specifier.

PART 1 GENERAL

Section Includes

Exterior high-speed coiling rubber doors.

Related Requirements

Section << 260583 - Wiring Connections>>:  Power to disconnect.

Reference Standards

ASTM A36/A36M - Standard Specification for Carbon Structural Steel; 2019.

ASTM A500/A500M - Standard Specification for Cold-Formed Welded and Seamless Carbon Steel Structural Tubing in Rounds and Shapes; 2023.

ASTM E330/E330M - Standard Test Method for Structural Performance of Exterior Windows, Doors, Skylights and Curtain Walls by Uniform Static Air Pressure Difference; 2014 (Reapproved 2021).

DASMA 403 - Specification for High Speed Doors and Grilles; 2024.

FBC TAS 201 - Impact Test Procedures; Testing Application Standard; 1994.

FBC TAS 202 - Criteria for Testing Impact and Non-Impact Resistant Building Envelope Components Using Uniform Static Air Pressure; Testing Application Standard; 1994.

ITS (DIR) - Directory of Listed Products; Current Edition.

UL (DIR) - Online Certifications Directory; Current Edition.

~~~~~ MASTER NOTE ~~~~~~
Do not request submittals if drawings sufficiently describe the products of this section or if proprietary specifying techniques are used. The review of submittals increases the possibility of unintended variations from contract documents, thereby increasing the design professional's liability.

- ~~~ END OF MASTER NOTE ~~~~

Submittals

Product Data:  Submit manufacturer's standard literature showing materials and details of construction and finish.<<  Include data on electrical operation.; or  None - N/A>>

Shop Drawings:  Indicate rough and actual opening dimensions, anchorage methods, hardware locations, and installation details.

Manufacturer's Instructions:  Indicate installation sequence and installation, adjustment, and alignment procedures.

Manufacturer's qualification statement.

Installer's qualification statement.

Maintenance Data:  Indicate << lubrication requirements and frequency;  periodic adjustments required; and  _______>>.

Specimen warranty.

Quality Assurance

Manufacturer Qualifications:  Company specializing in manufacturing products specified in this section, with at least << three; or  _____>> years of<<  documented;    _______; or  None - N/A>> experience.

Installer Qualifications:  Company specializing in performing work of type specified and with at least << three; or  _____>> years<<  documented;    _____; or  None - N/A>> experience<< and approved by manufacturer; or  None - N/A>>.

Products Requiring Electrical Connection:  Listed and classified by << ITS (DIR);  UL (DIR);  testing firm acceptable to authorities having jurisdiction; or  ___>> as suitable for purpose specified.

Delivery, Storage, and Handling

Store products in manufacturer's unopened packaging until ready for installation.

Protect materials from exposure to moisture. Do not deliver until after wet work complete and dry.

Store materials in dry, warm, ventilated weathertight location.

Warranty

Manufacturer Warranty:  Provide << 5-year; or  _______>> manufacturer warranty. Complete forms in <<Owner>>'s name and register with manufacturer.

PART 2 PRODUCTS

Manufacturer

- ~~~~~ MASTER NOTE ~~~~~~
Founded in 1954, Wayne Dalton was built on a dedication to ingenuity and customer service. As our business grew, so did our commitment to expanding our product lines, leading the company to become one of the largest door manufacturers in the United States. Since the beginning, we have remained committed to being the garage door partner whose innovation drives dealer productivity and provides industry-leading garage door solutions. In continuing this innovation, Wayne Dalton offers products, not only through our dealer connections, but also through our retail partner, Lowe's. Wayne Dalton offers a broad selection of residential garage doors, as well as a complete line of doors for commercial and industrial applications. Our Commercial Sectional Doors are ideal for jobs that call for excellent thermal efficiency, have stringent wind load requirements, or simply need a high performing door designed for long life and superior strength. Our Commercial Rolling Doors include service doors, counter doors, fire doors, security grilles, security shutters and roll up sheet doors. In 2016, we expanded our commercial door category to include High Speed Doors. Wayne Dalton designed and built a new series of High-Speed Doors from the ground up. These doors feature patent-pending, high-cycle designs, and industry-leading safety features for both interior and exterior applications. With everything from residential garage doors to rolling doors to commercial sectional to high performance doors, Wayne Dalton’s breadth of product can provide a garage door solution for any application.   Contact Information: Wayne Dalton 2501 S State Hwy 121 Bus, Suite 200 Lewisville, Texas 75067 United States   Phone: (800) 827-3667 Email: info@wayne-dalton.com   Contact for Design Professional: Customer Service Phone: (800) 827-3667 Email: info@wayne-dalton.com   Online CEU Courses   Request Lunch & Learn   https://www.wayne-dalton.com/   Plant Location: Dalton, Ohio; Mt. Hope, Ohio, Pensacola, Florida; Centralia, Washington.    
~~~ END OF MASTER NOTE ~~~~

Wayne Dalton; www.wayne-dalton.com; 1 (800) 827-3667.

Substitutions: Not permitted.

Exterior High-Speed Coiling Rubber Doors

Wayne Dalton; << Model 885; Model 885LH >>; ADV-Xtreme exterior low-headroom, high-speed rubber door.

Curtain Fabric:  Multi-reinforced nitrile butadiene rubber (NBR).

- ~~~~~ MASTER NOTE ~~~~~~
Manufacturer's standard maximum width:  22 feet (6706 mm).
~~~ END OF MASTER NOTE ~~~~

Width:  << ____ feet (____ mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Manufacturer's standard maximum height:  10 feet (3048 mm).
~~~ END OF MASTER NOTE ~~~~

Height:  << ____ feet (____ mm)>>.

Mounting:  Surface-mounted << on side indicated on drawings;  interior side;  exterior side; or  _______>>.

Performance Standards:  <<DASMA 403>> requirements for high-performance doors.

Opening Speed:  Door to operate at variable speed up to << 30 inches (762 mm)>> per second.

Closing Speed:  Door to operate at variable speed up to << 20 inches (508 mm)>> per second.

- ~~~~~ MASTER NOTE ~~~~~~
Availability: 12 inches (305 mm) for doors 8 to 14 feet (2438 to 4267 mm) wide. 13 inches (330 mm) for doors 15 to 18 feet (4572 to 5486 mm) wide. 15 inches (381 mm) for doors 19 to 22 feet (5791 to 6706 mm) wide.
~~~ END OF MASTER NOTE ~~~~

Headroom:  << 12 inches (305 mm);  13 inches (330 mm);  15 inches (381 mm); or  _____ inches (_____ mm)>>.

Operation Cycles:  Drive motor and gearbox capable of operating for not less than one million cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Durability:  Doors to operate at minimum of one million cycles when selected, installed, operated, and maintained.

- ~~~~~ MASTER NOTE ~~~~~~
Wind load shown is based on door width of 19 feet (5.8 m).
~~~ END OF MASTER NOTE ~~~~

Wind Load:  Design door assembly to withstand ultimate static wind load of << 27 psf (1293 Pa)>> in accordance with <<ASTM E330/E330M>>.

Curtain Fabric:  Five-layer NBR rubber with four layers of polyester weave.

- ~~~~~ MASTER NOTE ~~~~~~
Black and blue are both standard options.
~~~ END OF MASTER NOTE ~~~~

Fabric Color:  << Black;  Blue; or  _____>>.

Vision Panels:  << 20 inches (508 mm)>> high, spaced evenly across door.

Material:  Clear PVC.

Curtain Retainers:  Nylon 66 curtain lock at outside edges of curtain engaged inside guides under wind load.

Bottom Bar:  Double-angle bottom bar full width of opening, sufficient to maintain bottom edge of curtain parallel to door threshold.

- ~~~~~ MASTER NOTE ~~~~~~
Premium powder coat in safety yellow is standard finish. Other options include Premium in black, Premium RAL, and Premium custom color.
~~~ END OF MASTER NOTE ~~~~

Finish:  Premium powder coat in << safety yellow; or  _______>>.

Upon impact, bottom bar releases from guides and door operation stops.

Controller indicates problem encountered and instructs operator on steps to fix problem.

Detection required via solid-state device for accuracy; allow no external electromechanical switch.

Press button on control panel to reset door after impact.

Provide door with wireless failsafe electric safety edge.

Include field-adjustable breakaway sensitivity.

Weatherstripping and Seals:  Vinyl brush seal, resilient type, located at guide edges and where curtain enters hood enclosure. Vinyl-wrapped bottom seal, padded bottom bar.

Side Guides, Channels:  Constructed of high-strength steel with members fully bolted together. Guides have full-height weather seal.

- ~~~~~ MASTER NOTE ~~~~~~
Premium powder coat in safety yellow is standard. Other options include Premium in black, Premium RAL, and Premium custom color.
~~~ END OF MASTER NOTE ~~~~

Finish:   Premium powder coat in << safety yellow; or  _______>>.

Door Header:  Top-roll assembly fabricated of high-strength steel barrel, supported with powder-coated, high-strength steel brackets at each end with self-aligning bearings.

Drum Barrel System:  <<ASTM A500/A500M>> Grade B, high-strength steel pipe; minimum << 6-5/8-inch (168.3 mm)>> diameter.

Brackets:  <<ASTM A36/A36M>> hot-rolled steel with heavy-duty, self-aligning bearings with cast iron housings; minimum << 1/4 inch (6 mm)>> thick.

Springless System:  No balancing springs or counterweights permitted.

Head frame provided with single brush seal along lintel.

- ~~~~~ MASTER NOTE ~~~~~~
Premium powder coat in black is default finish. Other options include Premium RAL and Premium custom color.
~~~ END OF MASTER NOTE ~~~~

Finish:  Premium powder coat in << black; or  _______>>.

Hood:  << Top-roll assembly enclosed with external metal hood; or  Not required>>.

- ~~~~~ MASTER NOTE ~~~~~~
Black polyester topcoat is default finish. Other finish options include Premium RAL and Premium custom color.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Black polyester topcoat; or  _______>>.

Material:  Galvanized steel with intermediate supports.

- ~~~~~ MASTER NOTE ~~~~~~
Square is typical; sloped is recommended to shed water in exterior or washdown locations.
~~~ END OF MASTER NOTE ~~~~

Shape:  << Square;  Sloped; or  _______>>.

Motor Cover:  << Not required; or  Required>>.

Steel Finish:  <<  Premium powder coat;   Zinc base coat with  Premium topcoat;   Anti-microbial powder coat; or  _______>>.

Obstruction Safety Detection:

Dual infrared sensors mounted in-plane to door curtain at << 18 inches (457 mm)>> and << 50 to 54 inches (1270 to 1372 mm)>> from floor.

Light curtain in-plane to travelling path of door curtain. Size:  << 3 feet (914 mm);  6 feet (1829 mm); or  _____ feet (_____ mm)>>.

Reversing Safety Edge:  Door provided with monitored failsafe electric safety edge. Controller indicates inoperable safety edge.

Wireless connections between safety edge and controller required. No coil cords allowed.

Require ground level replacement of bottom bar wireless system battery.

Operation:  Direct drive motor and gearbox system with field-changeable universal handing and gearbox safety device.

Control Panel:  Variable frequency drive NEMA 4X cULus listed, onboard self-diagnostic and service reminder, built-in padlockable fused disconnect, and easy-to-read LCD text screen for troubleshooting.

Capable of monitoring and reporting on various operating conditions, including current operating status, current command status, current error status, hoist interlock status, service reminder status, and 24 VDC status.

Manual Override:  Pull switch and hand chain allowing manual door operation during power outage and installation.

Limit System:  Magnetic type providing absolute positioning with push-to-set and remote-setting capabilities. Limit system to synchronize with door during manual operation and supply power interruptions.

Timer to Close:  Each door to have automatic closing controlled by adjustable hold-open time delay.

- ~~~~~ MASTER NOTE ~~~~~~
If project requires a disconnect, select a three-phase power option. If door size is greater than 10 by 10 feet, select three-phase power option.
~~~ END OF MASTER NOTE ~~~~

Operation Supply Voltage:  << 208 VAC three-phase with transformer;  230 VAC three-phase;  460 VAC three-phase;  575 VAC three-phase with transformer; or  _____>>.

Actuation Device Options:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch;  radio control;  wireless connection; and  _______>>.

Signaling Device Option:  << Red and green LED warning lights;  Horn and strobe combination;  Rotating warning light; or  _______>>.

Optional LED Lighting System:  Programmable light bar << 3 feet (914 mm)>> tall with red, yellow, green lighting.

PART 3  EXECUTION

- ~~~~~ MASTER NOTE ~~~~~~
Include requirements for determining suitability of conditions for required work.   Include field testing requirements for materials, substrates, systems, etc.   Specify ambient condition requirements in PART 1 in the FIELD CONDITIONS article.
~~~ END OF MASTER NOTE ~~~~

Examination

Verify << existing conditions; and  _______>> meet manufacturer's requirements before starting work.

Verify opening sizes, tolerances and conditions are acceptable.

Installation

Install in accordance with manufacturer's written instructions.

Use anchorage devices to securely fasten assembly to wall construction and building structure without distortion or stress.

Securely and rigidly brace components suspended from structure.<<  Secure guides to structural members only.;    ______ .; or  None - N/A>>

Fit and align assembly including hardware; level and plumb, to provide smooth operation.

Coordinate installation of electrical service; see Section << 260583>>.

Install enclosure and perimeter trim.

- ~~~~~ MASTER NOTE ~~~~~~
Limit tolerances to installed dimensions, such as variation in thickness or deviation from intended locations.
~~~ END OF MASTER NOTE ~~~~

Tolerances

Maintain dimensional tolerances and alignment with adjacent work.

Maximum Variation from Plumb:  << 1/16 inch (1.6 mm); or  ___ inch (___ mm)>>.

Maximum Variation from Level:  << 1/16 inch (1.6 mm); or  ___ inch (___ mm)>>.

Longitudinal or Diagonal Warp:  Plus or minus << 1/8 inch per 10 feet (3.2 mm per 3 m); or  ____ inch per 10 feet (____ mm per 3 m)>>.

Adjusting

Adjust operating assemblies for smooth and noiseless operation.

Cleaning

See Section << 017000 - Execution and Closeout Requirements>> for additional requirements.

Clean installed components.

Remove labels and visible markings.

- ~~~~~ MASTER NOTE ~~~~~~
Describe protection of completed work.
~~~ END OF MASTER NOTE ~~~~

Protection

Protect finishes until completion of project.

Touch up damaged finishes after Date of Substantial Completion.

- END OF SECTION