Section 083613
Thermospan sectional Steel Doors
models TS-125, TS-150, Model 200, Model 200-20

NOTE: Enable Microsoft Word option to display hidden text to view notes to specifier.

PART 1 GENERAL

Section Includes

Upward acting sectional steel doors.

Related Requirements

Section << 079200 - Joint Sealants>>:  Sealing joints between frames and adjacent construction.

Section << 087100 - Door Hardware>>:  Cylinder cores and keys.

Section << 260533.13 - Conduit for Electrical Systems>>:  Conduit from electric circuit to operator and from operator to control station, and empty conduit from control units to door operator.

Section << 260583 - Wiring Connections>>:  Power to disconnect.

Definitions

DSB:  Double Strength Billet.

SSB:  Single Strength Billet.

Reference Standards

ITS (DIR) - Directory of Listed Products; Current Edition.

UL (DIR) - Online Certifications Directory; Current Edition.

UL 325 - Standard for Door, Drapery, Gate, Louver, and Window Operators and Systems; Current Edition, Including All Revisions.

~~~~~ MASTER NOTE ~~~~~~
Do not request submittals if drawings sufficiently describe the products of this section or if proprietary specifying techniques are used. The review of submittals increases the possibility of unintended variations from contract documents, thereby increasing the design professional's liability.

- ~~~ END OF MASTER NOTE ~~~~

Submittals

Product Data:  Submit manufacturer's standard literature showing materials and details of construction and finish.<<  Include data on electrical operation.; or  None - N/A>>

Shop Drawings:  Indicate rough and actual opening dimensions, anchorage methods, hardware locations, and installation details.

Manufacturer's Installation Instructions:  Indicate installation sequence and installation, adjustment, and alignment procedures.

Manufacturer's qualification statement.

Installer's qualification statement.

Operation and Maintenance Data:  Indicate modes of operation, lubrication requirements and frequency, and periodic adjustments required.

Specimen warranty.

Quality Assurance

Manufacturer Qualifications:  Company specializing in performing work of this section with minimum of << 5 years; or  _____ years>> of<<  documented;    _____; or  None - N/A>> experience in fabrication and installation of security closures.

Installer Qualifications:  Company specializing in performing work of this section with minimum of << 3 years; or  _____ years>> of<<  documented;    _____; or  None - N/A>> experience and approved by manufacturer.

Warranty

Manufacturer Door-Only Warranty:  Provide manufacturer warranty for door assembly for duration indicated under individual doors. Complete forms in <<Owner>>'s name and register with manufacturer.

Manufacturer Door and Operator Warranty:  Provide manufacturer's limited warranty for door and operator system free from material and workmanship defects for duration and cycles indicated under individual doors; counterbalance spring and finish not covered by warranty. Available on Thermospan models and Genie Commercial Operators.

PART 2 PRODUCTS

Manufacturer

- ~~~~~ MASTER NOTE ~~~~~~
Founded in 1954, Wayne Dalton was built on a dedication to ingenuity and customer service. As our business grew, so did our commitment to expanding our product lines, leading the company to become one of the largest door manufacturers in the United States. Since the beginning, we have remained committed to being the garage door partner whose innovation drives dealer productivity and provides industry-leading garage door solutions. In continuing this innovation, Wayne Dalton offers products, not only through our dealer connections, but also through our retail partner, Lowe's. Wayne Dalton offers a broad selection of residential garage doors, as well as a complete line of doors for commercial and industrial applications. Our Commercial Sectional Doors are ideal for jobs that call for excellent thermal efficiency, have stringent wind load requirements, or simply need a high performing door designed for long life and superior strength. Our Commercial Rolling Doors include service doors, counter doors, fire doors, security grilles, security shutters and roll up sheet doors. In 2016, we expanded our commercial door category to include High Speed Doors. Wayne Dalton designed and built a new series of High-Speed Doors from the ground up. These doors feature patent-pending, high-cycle designs, and industry-leading safety features for both interior and exterior applications. With everything from residential garage doors to rolling doors to commercial sectional to high performance doors, Wayne Dalton’s breadth of product can provide a garage door solution for any application.   Contact Information: Wayne Dalton 2501 S State Hwy 121 Bus, Suite 200 Lewisville, Texas 75067 United States   Phone: (800) 827-3667 Email: info@wayne-dalton.com   Contact for Design Professional: Customer Service Phone: (800) 827-3667 Email: info@wayne-dalton.com   Online CEU Courses   Request Lunch & Learn   https://www.wayne-dalton.com/   Plant Location: Dalton, Ohio; Mt. Hope, Ohio, Pensacola, Florida; Centralia, Washington.    
~~~ END OF MASTER NOTE ~~~~

Wayne Dalton; www.wayne-dalton.com; 1 (800) 827-3667.

Substitutions: Not permitted.

Upward Acting Sectional Steel Doors

Thermospan Steel Sectional Doors:  Wayne Dalton; << Model 125;  Model 150;  Model 200;  Model 200-20 >>.

Panels:  Steel construction with exterior sheet, interior sheet, and core reinforcement.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum widths:
Model 125: 18 feet 4 inches (5.58m)
Model 150: 26 feet 2 inches (7.99m)
Model 200-20: 24 feet 2 inches (7.38 m)
Model 200: 40 feet 2 inches (12.25 m)
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet _____ inches (_____ m)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum heights:
Models 125 and 200-20: 16 feet 1 inch (4.91 m)
Model 150: 20 feet 1 inch (6.13 m)
Model 200: 32 feet 1 inch (9.78 m)
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet _____ inches (_____ m)>>.

Mounting:  Surface mounted on interior of building.

Opening Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

Closing Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

- ~~~~~ MASTER NOTE ~~~~~~
Model 125: N/A
Model 200 and 200-20: 0.17 cfm/sq ft (3.11 cu m/hr/sq m) at 15mph
Model 150: 0.23 cfm/sq ft (4.21 cu m/hr/sq m) at 25mph
~~~ END OF MASTER NOTE ~~~~

Model 150:  Air Infiltration at << 25 mph (40 km/hr)>>:  << 0.08 cfm/sq ft (1.46 cu m/hr/sq m);  0.13 cfm/sq ft (2.37 cu m/hr/sq m);  0.15 cfm/sq ft (2.74 cu m/hr/sq m);  0.21 cfm/sq ft (3.83 cu m/hr/sq m);  0.46 cfm/sq ft (8.40 cu m/hr/sq m); or  4.21 cu m/hr/sq m (____ mm)>>.

Models 200 and 200-30:  Air Infiltration at << 15 mph (24 km/hr)>>:  << 0.17 cfm/sq ft (3.11 cu m/hr/sq m)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Model 125: 0.20
Model 150: 0.14
Models 200 and 200-20: 0.12
~~~ END OF MASTER NOTE ~~~~

U-Factor:  ​<< 0.12;  0.07;  0.14;  0.10; or  0.20>>​.

- ~~~~~ MASTER NOTE ~~~~~~
R-value (RSI-value):
Model 125: 10.79 (1.90).
Model 150: 14.16 (2.49).
Models 200-20 and 200: 17.5 (3.08).
~~~ END OF MASTER NOTE ~~~~

R-value (RSI-value):  ​<< 17.5 (3.08);  14.16 (2.49);  17.40 (3.09);  10.79 (1.90); or  9.31 (1.64)>>​.

- ~~~~~ MASTER NOTE ~~~~~~
Models 125: Class 21.
Models 150, 200 and 200-20: Class 22.
~~~ END OF MASTER NOTE ~~~~

STC Rating:  << Class 21; or  Class 22>>.

- ~~~~~ MASTER NOTE ~~~~~~
Select operation cycles to coordinate with selected springs.
~~~ END OF MASTER NOTE ~~~~

Operation Cycles:  Capable of operating for minimum << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door Only:  One year.

Panel Material:

- ~~~~~ MASTER NOTE ~~~~~~
Models 125 and 150: 0.012 inch (0.30 mm).
Model 200: 0.015 inch (0.38 mm).
Model 200-20: 20 gauge, 0.0355 inch (0.90 mm).
~~~ END OF MASTER NOTE ~~~~

Exterior Steel Thickness:  << 31-gauge, 0.012-inch (0.30 mm);  30-gauge, 0.0157-inch (0.40 mm);  22-gauge, 0.0336-inch (0.85 mm); or  21-gauge, 0.0366-inch (0.93 mm)>> galvanized steel.

- ~~~~~ MASTER NOTE ~~~~~~
Model 125: 7/8 inch (22 mm)
Model 150: 1-1/2 inch (38 mm)
Models 200 and 200-20: 2 inches (51 mm)
~~~ END OF MASTER NOTE ~~~~

Panel Thickness:  ​<< 2 inches (51 mm);  3 inches (76 mm);  1-1/2 inches (38 mm);  7/8 inch (22 mm); or  1-3/8 inches (35 mm)>>​.

Joint Profile:  Tongue and groove.

Section Height:  Manufacturer's standard based on door height.

- ~~~~~ MASTER NOTE ~~~~~~
Models 125 and 200: Flush; non-repeating, random stucco texture; or 1/4-inch wide pinstriping.
Model 150: Stucco texture or 1/4-inch wide pinstriping.
Models 200 and 200-20: Flush or non-repeating, random stucco texture.
~~~ END OF MASTER NOTE ~~~~

Exterior Surface Profile:  << Flush;  Ribbed;  Microgroove;  Stucco texture; or  Raised panel>>.

- ~~~~~ MASTER NOTE ~~~~~~
Models 125 and 150: Double thermal acrylic, aluminum sash section with DSB glazing, insulated DSB, or clear long lites.
Models 200 and 200-20: Double thermal acrylic, aluminum sash section with DSB glazing, insulated DSB, or clear long lites. Clear long lites not available on doors over 20 feet long.
~~~ END OF MASTER NOTE ~~~~

Lites:  << Not required;  Double thermal acrylic;  Aluminum sash section with double strength billet (DSB) glazing;  Insulated double strength billet (DSB);  Clear long lites; or  Large lites>>.

- ~~~~~ MASTER NOTE ~~~~~~
Model 200: White stucco, tan stucco, or brown stucco.
Model 150: White stucco, tan stucco, brown stucco, or gray stucco.
Models 125 and 200-20 standard finish color: White.
~~~ END OF MASTER NOTE ~~~~

Exterior Finish:  << White;  Tan;  Brown;  Gray;  Almond;  Industrial Brown;  Black;  Sandstone;  Forest Green;  Chestnut Brown;  Terra Bronze;  Desert Tan;  Trinar White; or  Trinar Brown>>.

Locking Options to Include:  << None;  Slide lock; or  Cylinder>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>>.

Weatherstripping and Seals:  << Required; or  Not required>>.

Tracks:  Constructed of steel with members fully bolted together.

Size:  << 2 inches (51 mm); or  3 inches (76 mm)>>.

Finish:  Mill finish.

- ~~~~~ MASTER NOTE ~~~~~~
Models 125 and 150: 0.0555 inch (1.410 mm)
Models 200 and 200-20: 0.0356 inch (0.904 mm)
~~~ END OF MASTER NOTE ~~~~

End Stiles:  ​<< 17-gauge, 0.0575-inch (1.46 mm);  14-gauge, 0.0785-inch (1.99 mm);  21-gauge, 0.0366-inch (0.93 mm);  single 16-gauge, 0.0635-inch  (1.613 mm); or  double 16-gauge, 0.0635-inch (1.613 mm)>>​ galvanized steel.

Brackets:  Steel to support counterbalance and curtain.

Finish:  Hot-dip galvanized.

Springs:  Rated for << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up to 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m). Chain hoist over 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m).
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Push-up; or  Chain hoist>>.

Motor Operation:  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction at minimum << 6 inches (152 mm)>> per second, maximum << 12 inches (305 mm)>> per second.

Timer to Close:  << Not required; or  Automatic closing controlled by adjustable hold-open time delay>>.

Operation Supply Voltage:  << 115/208/230V 1 phase 60Hz;     208/230/460V 3 phase 60Hz;     575V 3 phase 60Hz;     220V 1 phase 50Hz;     220V 3 phase 50Hz; or     400V 3 phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  << Wall mounted; or  Trolley>>, << right-hand side;  left-hand side; or  center>>.

Motor Enclosure:  << Not required;  NEMA 4-12 watertight and oiltight;  NEMA 4X corrosion-resistant;  Totally enclosed, fan-cooled motor; or  Totally enclosed, nonventilated>>.

Obstruction Safety Detection:  Infrared sensor mounted in-plane to door curtain at << 6 inches (152 mm)>> from floor.

Reversing Safety Edge:  << Not required;  Door provided with wireless monitored electric safety edge; or  Door provided with wired monitored electric safety edge>>.

PART 3 EXECUTION

- ~~~~~ MASTER NOTE ~~~~~~
Include requirements for determining suitability of conditions for required work.   Include field testing requirements for materials, substrates, systems, etc.   Specify ambient condition requirements in PART 1 in the FIELD CONDITIONS article.
~~~ END OF MASTER NOTE ~~~~

Examination

Verify wall openings are ready to receive work and opening dimensions and tolerances are within specified limits.

Verify electric power is available and meets manufacturer's requirements.

Preparation

Prepare opening to permit correct installation of door unit to perimeter air and vapor barrier seal.

Apply primer to wood frame.

Installation

Install door unit assembly in accordance with manufacturer's instructions.

Anchor assembly to wall construction and building framing without distortion or stress.

Securely brace door tracks suspended from structure. Secure tracks to structural members only.

Fit and align door assembly, including hardware.

Coordinate installation of electrical service. Complete power and control wiring from disconnect to unit components.

Install << perimeter trim;  closures; and  _____>>.

- ~~~~~ MASTER NOTE ~~~~~~
Limit tolerances to installed dimensions, such as variation in thickness or deviation from intended locations.
~~~ END OF MASTER NOTE ~~~~

Tolerances

Maximum Variation from Plumb:  << 1/16 inch (1.5 mm); or  ___ inch (___ mm)>>.

Maximum Variation from Level:  << 1/16 inch (1.5 mm); or  ___ inch (___ mm)>>.

Longitudinal or Diagonal Warp:  Plus or minus << 1/8 inch in 10 feet (3 mm in 3 m); or  _____ inch in 10 feet (_____ mm in 3 m)>> straight edge.

Maintain dimensional tolerances and alignment with adjacent work.

Adjusting

Adjust door assembly for smooth operation and full contact with weatherstripping.

Confirm operation is in accordance with manufacturer's instructions with present field representative. Identify adjustments to door assembly.

Cleaning

See Section << 017000 - Execution and Closeout Requirements>> for additional requirements.

Clean doors and frames<<  and glazing;    _____; or  None - N/A>>.

Remove<<  temporary;    _____; or  None - N/A>> labels and visible markings.

- ~~~~~ MASTER NOTE ~~~~~~
Describe protection of completed work.
~~~ END OF MASTER NOTE ~~~~

Protection

Protect installed products from damage until Date of Substantial Completion.

Do not permit construction traffic through overhead door openings after adjustment and cleaning.

- END OF SECTION