Section 083323
upward coiling Doors
model 800c hc

NOTE: Enable Microsoft Word option to display hidden text to view notes to specifier.

PART 1 GENERAL

Section Includes

Upward coiling service doors.

Related Requirements

Section << 079200 - Joint Sealants>>:  Sealing joints between frames and adjacent construction.

Section << 083313 - Coiling Counter Doors>>:  Additional types of overhead coiling doors.

Section << 087100 - Door Hardware>>:  Cylinder cores and keys.

Section << 260583 - Wiring Connections>>:  Power to disconnect.

Reference Standards

ASHRAE Std 90.1 I-P - Energy Standard for Buildings Except Low-Rise Residential Buildings; Most Recent Edition Adopted by Authority Having Jurisdiction, Including All Applicable Amendments and Supplements.

ASTM A653/A653M - Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process; 2023.

ASTM E90 - Standard Test Method for Laboratory Measurement of Airborne Sound Transmission Loss of Building Partitions and Elements; 2023.

ASTM E330/E330M - Standard Test Method for Structural Performance of Exterior Windows, Doors, Skylights and Curtain Walls by Uniform Static Air Pressure Difference; 2014 (Reapproved 2021).

FBC TAS 201 - Impact Test Procedures; Testing Application Standard; 1994.

FBC TAS 202 - Criteria for Testing Impact and Non-Impact Resistant Building Envelope Components Using Uniform Static Air Pressure; Testing Application Standard; 1994.

ICC (IECC)-2018 - International Energy Conservation Code; 2018.

UL (DIR) - Online Certifications Directory; Current Edition.

~~~~~ MASTER NOTE ~~~~~~
Do not request submittals if drawings sufficiently describe the products of this section or if proprietary specifying techniques are used. The review of submittals increases the possibility of unintended variations from contract documents, thereby increasing the design professional's liability.

- ~~~ END OF MASTER NOTE ~~~~

Submittals

Product Data:  Submit manufacturer's standard literature showing materials and details of construction and finish.<<  Include data on electrical operation.; or  None - N/A>>

Shop Drawings:  Indicate rough and actual opening dimensions, anchorage methods, hardware locations, and installation details.

Manufacturer's Instructions:  Indicate installation sequence and installation, adjustment, and alignment procedures.

Manufacturer's qualification statement.

Installer's qualification statement.

Operation and Maintenance Data:  Indicate modes of operation, lubrication requirements and frequency, and periodic adjustments required.

Specimen warranty.

Quality Assurance

Manufacturer Qualifications:  Company specializing in performing work of this section with minimum of << 5 years; or  _____ years>> of<<  documented;    _____; or  None - N/A>> experience in fabrication and installation of security closures.

Installer Qualifications:  Company specializing in performing work of this section with minimum of << 3 years; or  _____ years>> of<<  documented;    _____; or  None - N/A>> experience and approved by manufacturer.

Products Requiring Electrical Connection:  Listed and classified by << UL (DIR);  TUV;  testing firm acceptable to authorities having jurisdiction; or  _____>> as suitable for purpose specified.

Delivery, Storage, and Handling

Store products in manufacturer's unopened packaging until ready for installation.

Protect materials from exposure to moisture.

Store materials in dry, warm, ventilated, weathertight location.

Warranty

Manufacturer Door and Operator Warranty:  Provide manufacturer's limited warranty for door and operator system free from material and workmanship defects for duration and cycles indicated under individual doors; counterbalance spring and finish not covered by warranty.

PART 2 PRODUCTS

Manufacturer

- ~~~~~ MASTER NOTE ~~~~~~
Founded in 1954, Wayne Dalton was built on a dedication to ingenuity and customer service. As our business grew, so did our commitment to expanding our product lines, leading the company to become one of the largest door manufacturers in the United States. Since the beginning, we have remained committed to being the garage door partner whose innovation drives dealer productivity and provides industry-leading garage door solutions. In continuing this innovation, Wayne Dalton offers products, not only through our dealer connections, but also through our retail partner, Lowe's. Wayne Dalton offers a broad selection of residential garage doors, as well as a complete line of doors for commercial and industrial applications. Our Commercial Sectional Doors are ideal for jobs that call for excellent thermal efficiency, have stringent wind load requirements, or simply need a high performing door designed for long life and superior strength. Our Commercial Rolling Doors include service doors, counter doors, fire doors, security grilles, security shutters and roll up sheet doors. In 2016, we expanded our commercial door category to include High Speed Doors. Wayne Dalton designed and built a new series of High-Speed Doors from the ground up. These doors feature patent-pending, high-cycle designs, and industry-leading safety features for both interior and exterior applications. With everything from residential garage doors to rolling doors to commercial sectional to high performance doors, Wayne Dalton’s breadth of product can provide a garage door solution for any application.   Contact Information: Wayne Dalton 2501 S State Hwy 121 Bus, Suite 200 Lewisville, Texas 75067 United States   Phone: (800) 827-3667 Email: info@wayne-dalton.com   Contact for Design Professional: Customer Service Phone: (800) 827-3667 Email: info@wayne-dalton.com   Online CEU Courses   Request Lunch & Learn   https://www.wayne-dalton.com/   Plant Location: Dalton, Ohio; Mt. Hope, Ohio, Pensacola, Florida; Centralia, Washington.    
~~~ END OF MASTER NOTE ~~~~

Wayne Dalton; www.wayne-dalton.com; 1 (800) 827-3667.

Substitutions: Not permitted.

Upward Coiling Service Doors

Wayne Dalton; Model 800C HC Service Door.

Slats:  Insulated, interlocking, roll-formed, metal slats.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 20 feet (6.1 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 20 feet (6.1 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _______>>.

Mounting:  Surface mounted on side indicated on drawings.

Opening Speed:  Door to operate at variable speed up to << 8 to 12 inches (203 to 305 mm)>> per second.

Closing Speed:  Door to operate at variable speed up to << 8 to 12 inches (203 to 305 mm)>> per second.

Operation Cycles:  Capable of operating for minimum 500,000 cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Limited Warranty:  Three years or 500,000 cycles, whichever comes first.

- ~~~~~ MASTER NOTE ~~~~~~
Maximum door widths for available wind load ratings:
20 psf: 20 feet (6096 mm)
31 psf: 33 feet (10,058 mm); impact rated available up to 22 feet
37 psf: 28 feet (8534 mm); impact rated available up to 22 feet 
44 psf: 10 feet (3048 mm)
47 psf: 22 feet (6705 mm); impact rated available up to 22 feet
53 psf: 19 feet (5791 mm); impact rated available 
55 psf: 22 feet (6705 mm); impact rated available 
~~~ END OF MASTER NOTE ~~~~

Wind Load:  Design door assembly to withstand ultimate static pressure load of << 20 psf (0.96 kPa);  31 psf (1.48 kPa);  37 psf (1.77 kPa);  44 psf (2.11 kPa);  47 psf (2.25 kPa);  50 psf (2.39 kPa);  53 psf (2.54 kPa);  55 psf (2.63 kPa); or  56 psf (2.68 kPa)>> in accordance with <<ASTM E330/E330M>>.

Wind Load:  Not required.

- ~~~~~ MASTER NOTE ~~~~~~
Impact Rating Requirements:
Dade County: 47 psf (2.3 kPa) and 55 psf (2.63 kPa)
FBC HVHZ: 47 psf (2.3 kPa) and 55 psf (2.63 kPa)
~~~ END OF MASTER NOTE ~~~~

Impact Rating:  Available with certification for large missile impact resistance (<<FBC TAS 201>> and <<FBC TAS 202>>), with design pressure of << _____ psf (_____ Pa)>> at << _____ feet (_____ mm)>> wide.<<  This design is approved by the Florida Building Commission, FL # _____ .;    This design is approved by Miami Dade NOA,; or  None - N/A>>

Curtain Material:

Galvanized Steel:  << 18 gauge, 0.0516 inch (1.31 mm);  20 gauge, 0.0396 inch (1.01 mm);  22 gauge, 0.0336 inch (0.86 mm);  24 gauge, 0.0278 inch (0.63 mm); or  ____ gauge, _____ inch (_____ mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Powder coat available in approximately 200 colors; custom color match options available.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Galvanized, primed white;  Galvanized, primed gray;  Galvanized, primed beige;  Galvanized, primed brown;  Galvanized G90 (Z275);  Galvanized G90 (Z275), primed gray; or  Powder coat; _____ color>>.

Aluminum:  << 0.040 inch (1.02 mm); or  _____ inch (_____ mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Powder coat available in approximately 200 colors; custom color match options available.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Mill finish;  Clear anodized;  Bronze anodized; or  Powder coat; _____ color>>.

Stainless Steel:  << 18 gauge, 0.0478 inch (1.27 mm);  20 gauge, 0.0359 inch (0.91 mm);  22 gauge, 0.0299 inch (0.76 mm); or  _____ gauge, _____ inch (_____ mm)>>.

Finish:  << No.2B mill; or  No.4 satin>>.

Slat Profile:  No.34, << 3-inch (76 mm)>> tall flat slat.

Slat R-Value:  << R-value (RSI-value):  7.7 (1.35)>>.

Installed System Sound Rating:  STC-21 in accordance with <<ASTM E90>>.

Vision Lites:  << Not required;  3/4 by 5 inches (19 by 127 mm) with Plexiglas covers; or  1 by 10 inches (25 by 254 mm) with Plexiglas covers>>.

Bottom Bar:  << 6063-T6 aluminum;  Galvanized steel; or  304 Stainless steel>>.

- ~~~~~ MASTER NOTE ~~~~~~
Double angle available for galvanized steel, stainless steel, and aluminum.
Extruded tube available for aluminum only.
~~~ END OF MASTER NOTE ~~~~

Profile:  Double angle.

Finish:  << Mill finish;  Clear anodized;  Bronze anodized;  Hot-dip galvanized;  Galvanized, primed black;  Powder coat; or  Stainless No.4 satin>>.

Color:  << Match curtain;  Black; or  _______>>.

Weatherstripping and Seals:

Bottom Seal Astragal:  Included.

Exterior Guide Seal:  << Required; or  Not required>>.

Interior Guide Seal:  << Required; or  Not required>>.

Internal Hood Seal:  Included.

Lintel Brush Seal:  << Required; or  Not required>>.

Side Guides, Channels:  Constructed of << steel; or  304 stainless steel>> with members fully bolted together.

Finish:  << Hot-dip galvanized;  Galvanized, primed black;  PowderGuard Premium powder coat;  Stainless No.4 satin;  PowderGuard Textured powder coat;  Powder coat; or  PowderGuard Zinc powder coat>>.

Color:  << Match curtain;  _______; or  Black>>.

Brackets:  Steel to support counterbalance and curtain.

Finish:  << Hot-dip galvanized;  Galvanized, primed black; or  Powder coat>>.

Color:  << Match curtain; or  _____>>.

Door Roll:  Directly driven, springless roll constructed of steel tube with integral shafts, keyed on drive end and supported by self-aligning, greasable, sealed bearings. Counterbalance device not required.

Hood:  << Not required;  24 gauge, 0.024 inch stainless steel (0.61 mm stainless steel);  24 gauge, 0.028 inch galvanized steel (0.63 mm galvanized steel);  22 gauge, 0.025 inch B&S aluminum (0.64 mm B&S aluminum); or  _____ gauge, _____ inch (_____ mm)>>.

Finish:  << Galvanized, primed to match curtain;  Hot-dip galvanized;  Clear anodized;  Bronze anodized;  Mill finish;  Powder coat; _____ color; or  Stainless No.4 satin>>.

Motor Operation:  Direct drive, integrated gear motor and brake assembly; size as recommended by manufacturer to move door in either direction at minimum << 8 inches (203 mm)>>, maximum << 1 foot (305 mm)>> per second. Operator and drive assembly is factory preassembled and provided with wiring harnesses needed direct from factory.

Manual Override:  Hoist.

Timer to Close:  Time-delay self-close timer and non-resettable cycle counter.

Operation Supply Voltage:  << 208V 1-phase 60Hz;  208V 3-phase 60Hz;  230V 3-phase 60Hz;  575V 3-phase 60Hz;  220-240V 1 phase 50Hz; or  440-480V 3 phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  Shaft mount.

Obstruction Safety Detection:

Primary, Monitored:  << Reflective photo-eyes;  Thru-beam photo-eyes;  Dual photo-eyes; or  2-wire reversing safety edge>>.

Secondary, Non-Monitored:  << Reflective photo-eyes;  Thru-beam photo-eyes;  Dual photo-eyes; or  2-wire reversing safety edge>>.

Control Panel:  Electronic controller with microprocessor self-diagnostics. Digital readout indicates door action, alarm conditions, and fault conditions. Enclosure is IP54-rated, NEMA 3 equivalent.

- ~~~~~ MASTER NOTE ~~~~~~
Model 800C ADV door has motor with 50 times the operating cycles and more than 3 times the speed.
~~~ END OF MASTER NOTE ~~~~

PART 3 EXECUTION

Examination

Verify << existing conditions; and  _______>> meet manufacturer's requirements before starting work.

Verify opening sizes, tolerances, and conditions are acceptable.

Installation

Install in accordance with manufacturer's written instructions.

Use anchorage devices to securely fasten assembly to wall construction and building structure without distortion or stress.

Securely and rigidly brace components suspended from structure.<<  Secure guides to structural members only.;    _____ .; or  None - N/A>>

Fit and align assembly, including hardware. Level and plumb to provide smooth operation.

Coordinate installation of electrical service; see Section << 260583>>.

Install enclosure and perimeter trim.

Tolerances

Maintain dimensional tolerances and alignment with adjacent work.

Maximum Variation from Plumb:  << 1/16 inch (1.6 mm); or  ___ inch (___ mm)>>.

Maximum Variation from Level:  << 1/16 inch (1.6 mm); or  ___ inch (___ mm)>>.

Longitudinal or Diagonal Warp:  Plus or minus << 1/8 inch per 10 feet (3.2 mm per 3 m); or  ___ inch per 10 feet (___ mm per 3 m)>> straight edge.

Adjusting

Adjust operating assemblies for smooth and noiseless operation.

Cleaning

See Section << 017000 - Execution and Closeout Requirements>> for additional requirements.

Clean installed components.

Remove labels and visible markings.

Protection

Protect finishes until completion of project.

Touch up damaged finishes after Date of Substantial Completion.

- END OF SECTION