Section 083323
upward coiling Doors

NOTE: Enable Microsoft Word option to display hidden text to view notes to specifier.

PART 1 GENERAL

Section Includes

Upward coiling service doors.

High-performance springless upward coiling service doors.

Upward coiling security shutters.

Upward coiling sheet doors.

Related Requirements

Section << 079200 - Joint Sealants>>:  Sealing joints between frames and adjacent construction.

Section << 083313 - Coiling Counter Doors>>:  Additional types of overhead coiling doors.

Section << 087100 - Door Hardware>>:  Cylinder cores and keys.

Section << 260583 - Wiring Connections>>:  Power to disconnect.

Reference Standards

ASHRAE Std 90.1 I-P - Energy Standard for Buildings Except Low-Rise Residential Buildings; Most Recent Edition Adopted by Authority Having Jurisdiction, Including All Applicable Amendments and Supplements.

ASTM A653/A653M - Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process; 2023.

ASTM E90 - Standard Test Method for Laboratory Measurement of Airborne Sound Transmission Loss of Building Partitions and Elements; 2023.

ASTM E330/E330M - Standard Test Method for Structural Performance of Exterior Windows, Doors, Skylights and Curtain Walls by Uniform Static Air Pressure Difference; 2014 (Reapproved 2021).

FBC TAS 201 - Impact Test Procedures; Testing Application Standard; 1994.

FBC TAS 202 - Criteria for Testing Impact and Non-Impact Resistant Building Envelope Components Using Uniform Static Air Pressure; Testing Application Standard; 1994.

ICC (IECC)-2018 - International Energy Conservation Code; 2018.

UL (DIR) - Online Certifications Directory; Current Edition.

~~~~~ MASTER NOTE ~~~~~~
Do not request submittals if drawings sufficiently describe the products of this section or if proprietary specifying techniques are used. The review of submittals increases the possibility of unintended variations from contract documents, thereby increasing the design professional's liability.

- ~~~ END OF MASTER NOTE ~~~~

Submittals

Product Data:  Submit manufacturer's standard literature showing materials and details of construction and finish.<<  Include data on electrical operation.; or  None - N/A>>

Shop Drawings:  Indicate rough and actual opening dimensions, anchorage methods, hardware locations, and installation details.

Manufacturer's Instructions:  Indicate installation sequence and installation, adjustment, and alignment procedures.

Manufacturer's qualification statement.

Installer's qualification statement.

Operation and Maintenance Data:  Indicate modes of operation, lubrication requirements and frequency, and periodic adjustments required.

Specimen warranty.

Quality Assurance

Manufacturer Qualifications:  Company specializing in performing work of this section with minimum of << 5 years; or  _____ years>> of<<  documented;    _____; or  None - N/A>> experience in fabrication and installation of security closures.

Installer Qualifications:  Company specializing in performing work of this section with minimum of << 3 years; or  _____ years>> of<<  documented;    _____; or  None - N/A>> experience and approved by manufacturer.

Products Requiring Electrical Connection:  Listed and classified by << UL (DIR);  TUV;  testing firm acceptable to authorities having jurisdiction; or  _____>> as suitable for purpose specified.

Delivery, Storage, and Handling

Store products in manufacturer's unopened packaging until ready for installation.

Protect materials from exposure to moisture.

Store materials in dry, warm, ventilated, weathertight location.

Warranty

Manufacturer Door-Only Warranty:  Provide manufacturer warranty for door assembly for duration indicated under individual doors. Complete forms in <<Owner>>'s name and register with manufacturer.

Manufacturer Door and Operator Warranty:  Provide manufacturer's limited warranty for door and operator system free from material and workmanship defects for duration and cycles indicated under individual doors; counterbalance spring and finish not covered by warranty.

PART 2 PRODUCTS

Manufacturer

- ~~~~~ MASTER NOTE ~~~~~~
Founded in 1954, Wayne Dalton was built on a dedication to ingenuity and customer service. As our business grew, so did our commitment to expanding our product lines, leading the company to become one of the largest door manufacturers in the United States. Since the beginning, we have remained committed to being the garage door partner whose innovation drives dealer productivity and provides industry-leading garage door solutions. In continuing this innovation, Wayne Dalton offers products, not only through our dealer connections, but also through our retail partner, Lowe's. Wayne Dalton offers a broad selection of residential garage doors, as well as a complete line of doors for commercial and industrial applications. Our Commercial Sectional Doors are ideal for jobs that call for excellent thermal efficiency, have stringent wind load requirements, or simply need a high performing door designed for long life and superior strength. Our Commercial Rolling Doors include service doors, counter doors, fire doors, security grilles, security shutters and roll up sheet doors. In 2016, we expanded our commercial door category to include High Speed Doors. Wayne Dalton designed and built a new series of High-Speed Doors from the ground up. These doors feature patent-pending, high-cycle designs, and industry-leading safety features for both interior and exterior applications. With everything from residential garage doors to rolling doors to commercial sectional to high performance doors, Wayne Dalton’s breadth of product can provide a garage door solution for any application.   Contact Information: Wayne Dalton 2501 S State Hwy 121 Bus, Suite 200 Lewisville, Texas 75067 United States   Phone: (800) 827-3667 Email: info@wayne-dalton.com   Contact for Design Professional: Customer Service Phone: (800) 827-3667 Email: info@wayne-dalton.com   Online CEU Courses   Request Lunch & Learn   https://www.wayne-dalton.com/   Plant Location: Dalton, Ohio; Mt. Hope, Ohio, Pensacola, Florida; Centralia, Washington.    
~~~ END OF MASTER NOTE ~~~~

Wayne Dalton; www.wayne-dalton.com; 1 (800) 827-3667.

Substitutions: Not permitted.

Upward Coiling Service Doors

Wayne Dalton; Model 926 Rolling Service Door.

Slats:  Interlocking, roll-formed, galvanized steel slats.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 12 feet (3.66 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 12 feet (3.66 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _______>>.

Mounting:  << Surface-mounted on side indicated on drawings; or  Between jamb>>.

Opening Speed:  Door to operate at variable speed up to << 8 inches (203 mm)>> per second.

Closing Speed:  Door to operate at variable speed up to << 12 inches (305 mm)>> per second.

Operation Cycles:  Capable of operating for minimum 20,000 cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door Only:  Two years.

Door and Operator Warranty:  Three years or 20,000 cycles, whichever comes first.

Wind Load:  Design door assembly to withstand ultimate static pressure load of << 20 psf (0.96 kPa)>>.

Curtain Material:  << 26-gauge, 0.217-inch (0.55 mm)>> galvanized steel. Staking lock system locks end of each slat from lateral movement.

Slat Profile:  No.17, << 2-inch (51 mm)>> flat crown profile.

Finish:  Standard polyester base coat.

Color:  White.

Bottom Bar:  Steel.

Profile:  Double angle.

Finish:  << Hot-dip galvanized;  Mill finish;  Galvanized, primed black;  Bronze anodized; or  PowderGuard Premium powder coat; _____ color>>.

Locking Options to Include:  << None;  Slide lock;  Cylinder; or  Best cylinder>>.

Lock Locations:

Lock Access:  Coil side.

Weatherstripping and Seals:

Bottom Seal Astragal:  << Required; or  Not required>>.

Lintel Brush Seal:  << Required; or  Not required>>.

Side Guides, Channels:  Constructed of steel with members fully bolted together.

Finish:  << Galvanized; or  Powder coated; _____ color>>.

Brackets:  Steel to support counterbalance and curtain.

Finish:  << Galvanized; or  Powder coated; _____ color>>.

Counterbalance:  Helical torsion spring type, housed in steel tube or pipe barrel and supporting curtain with deflection limited to << 0.03 inch per foot (1:400 mm/sec)>> of span. Adjustable spring tension required.

Hood:  << Not required; or  24-gauge, 0.0278-inch galvanized steel  (0.63 mm galvanized steel)>>.

Finish:  << Match curtain;  Standard polyester base coat; white color;  Standard polyester base coat; brown color; or  PowderGuard Premium powder coat; _____ color>>.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up available up to 8 by 8 feet (2438 by 2438 mm). Chain hoist over 8 by 8 feet (2438 by 2438 mm).
Bracket-mounted crank available up to 12 feet 4 inches (3759 mm) in height or 10 feet 4 inches (3149 mm) in width.
Wall crank not available if between-jamb mounted; available up to 16 feet 4 inches (4978 mm) in width or 14 feet 4 inches (4369 mm) in height.
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Push-up;  Chain hoist;  Bracket-mounted crank; or  Wall-mounted crank>>.

Motor Operation:  << Not required; or  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction at minimum 8 inches (203 mm), maximum 1 foot (305 mm) per second>>.

Manual Override:  << Hoist;  Hoist and jackshaft release; or  Jackshaft release>>.

Timer to Close:  << Not required; or  Automatic closing controlled by adjustable hold-open time delay>>.

Operation Supply Voltage:  << 115/208/230V 1-phase 60Hz;  208/230/460V 3-phase 60Hz;  575V 3-phase 60Hz;  220V 1-phase 50Hz;  220V 3-phase 50Hz; or  400V 3-phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  << Wall-mounted; or  Front of hood>>, << right-hand side; or  left-hand side>>.

Motor Enclosure:  << Not required;  NEMA 4-12 watertight and oiltight;  NEMA 4X corrosion-resistant;  Totally enclosed, fan-cooled motor; or  Totally enclosed, non-ventilated>>.

Obstruction Safety Detection:  Constant contact button only.

Obstruction Safety Detection:

Primary, Monitored:

Photo Eyes:  << Reflective photo-eyes;  Thru-beam photo-eyes; or  NEMA 4X photo-eyes>>.

Light Curtain:  << 3 feet (0.9 m);  6 feet (1.8 m); or  _____ feet (_____ m)>>.

Safety Edge:  << 2-wire reversing safety edge;  Wireless reversing safety edge; or  ______>>.

Secondary, Non-Monitored:

Photo Eyes:  << Reflective photo-eyes;  Thru-beam photo-eyes; or  _____>>.

Light Curtain:  << 3 feet (0.9 m);  6 feet (1.8 m); or  _____ feet (_____ m)>>.

Safety Edge:  << 2-wire reversing safety edge; or  ______>>.

Wayne Dalton; Model 900 Rolling Service Door.

Slats:  Interlocking, roll-formed, metal slats.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 14 feet (4.27 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 14 feet (4.27 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _______>>.

Mounting:  << Surface-mounted on side indicated on drawings; or  Between jamb>>.

Opening Speed:  Door to operate at variable speed up to << 6 to 12 inches (152 to 305 mm)>> per second.

Closing Speed:  Door to operate at variable speed up to << 6 to 12 inches (152 to 305 mm)>> per second.

Operation Cycles:  Capable of operating for minimum << 20,000;  50,000;  100,000; or  200,000>> cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door Only:  Two years.

Door and Operator Warranty:  Three years or 20,000 cycles, whichever comes first.

~~~~~ MASTER NOTE ~~~~~~
31 psf (1.5 kPa) wind load rating available only for 22-gauge steel doors up to 10 feet (3 m) in width, maximum. 20 psf (0.96 kPa) available for 22-gauge doors over 10 feet (3 m) wide.
24- and 26-gauge steel doors only available with 20 psf (0.96 kPa) wind load rating.
~~~ END OF MASTER NOTE ~~~~

Wind Load:  Design door assembly to withstand ultimate static pressure load of << 20 psf (0.96 kPa);  31 psf (1.48 kPa);  26 psf (1.25 kPa);  36 psf (1.72 kPa);  41 psf (1.96 kPa);  50 psf (2.39 kPa);  57 to 130 psf (2.73 to 6.22 kPa);  65 psf (3.11 kPa); or  68.9 psf (3.29 kPa)>>.

Curtain Material:  Galvanized steel. Staking lock system locks end of each slat from lateral movement.

~~~~~ MASTER NOTE ~~~~~~
Curved profile type C-187, 1.87-inch tall slat, for doors up to 15 feet 4 inches (4.67 m) wide, fabricated of: 1) 24 gauge galvanized steel.
2) 22 gauge galvanized steel.
3) 20 gauge galvanized steel.
4) 18 gauge galvanized steel.
5) 22 gauge stainless steel.
6) 20 gauge stainless steel.
7) 0.040 inch (1 mm) aluminum. Curved profile type C-275, 2.75-inch tall slat, for doors up to and between 15 feet 4 inches (4.67 m) and 18 feet 4 inches (5.59 m) wide, fabricated of: 1) 22 gauge galvanized steel.
2) 20 gauge galvanized steel.
3) 18 gauge galvanized steel.
4) 16 gauge galvanized steel.
5) 22 gauge stainless steel.
6) 20 gauge stainless steel.
7) 0.050 inch (1.29 mm) aluminum. Curved profile type C-275 for doors between 18 feet 4 inches (5.59 m) and 25 feet 4 inches (7.72 m) wide, fabricated of: 1) 20 gauge galvanized steel.
2) 18 gauge galvanized steel.
3) 16 gauge galvanized steel.
4) 20 gauge stainless steel.
5) 0.064 inch (1.63 mm) aluminum. Curved profile type C-275 for doors between 25 feet 4 inches (7.72 m) and 40 feet (12.19 m) wide, fabricated of: 1) 18 gauge galvanized steel.
2) 16 gauge galvanized steel.
3) 0.064 inch (1.63 mm) aluminum. Maximum width is 27 feet. Flat profile type F-265, 2.65-inch tall slat, for doors up to 18 feet 4 inches (5.59 m) wide, fabricated of: 1) 22 gauge galvanized steel.
2) 20 gauge galvanized steel.
3) 18 gauge galvanized steel.
4) 16 gauge galvanized steel.
5) 22 gauge stainless steel.
6) 20 gauge stainless steel.
7) 0.040 inch (1 mm) aluminum. Flat profile type F-265 for doors between 18 feet 4 inches (5.59 m) and 25 feet 4 inches (7.72 m) wide, fabricated of: 1) 20 gauge galvanized steel.
2) 18 gauge galvanized steel.
3) 16 gauge galvanized steel.
4) 20 gauge stainless steel.
5) 0.050 inch (1.29 mm) aluminum. Flat profile type F-265 for doors between 25 feet 4 inches (7.72 m) and 40 feet (12.19 m) wide, fabricated of: 1) 18 gauge galvanized steel.
2) 16 gauge galvanized steel.
3) 0.050 inch (1.29 mm) aluminum. Maximum width is 27 feet.
~~~ END OF MASTER NOTE ~~~~

Slat Profile:  << No.2, 2-inch (51 mm) curved profile;  No.17, 2-inch (51 mm) flat profile;  F-265; or  FE-265>>.

- ~~~~~ MASTER NOTE ~~~~~~
Size 1 by 10 inches (25 by 254 mm) on F-265 slat only.
~~~ END OF MASTER NOTE ~~~~

Fenestrated Slats:  << Not required;  Secur-Vent perforated slat;  5/8 by 3 inches (16 by 76 mm);  1 by 10 inches (25 by 254 mm);  5/8 by 3 inches (16 by 76 mm) with Plexiglass covers; or  1 by 10 inches (25 by 254 mm) with Plexiglass covers>>.

Finish:

Standard polyester base coat; << gray;  beige;  white; or  brown>> color.

G-90 (Z275) galvanized<< ; primed gray; or  None - N/A>>.

Bottom Bar:  Steel.

- ~~~~~ MASTER NOTE ~~~~~~
Galvanized Steel: Double angle, single angle.
Stainless Steel: Double angle.
Aluminum: Double angle, extruded T, extruded tube.
~~~ END OF MASTER NOTE ~~~~

Profile:  Double angle.

Finish:  << Galvanized, primed black;  Black anodized;  Bronze anodized;  Hot-dip galvanized;  PowderGuard Premium powder coat;  PowderGuard Textured powder coat;  PowderGuard Zinc powder coat;  Stainless No.4 satin; or  Stainless No.2B mill>>.

Locking Options to Include:  << None;  Slide lock;  Cylinder; or  Best cylinder>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>>.

Lock Access:  << Coil side; or  Opposite coil side>>.

Weatherstripping and Seals:

- ~~~~~ MASTER NOTE ~~~~~~
Bottom seal astragal comes standard.
~~~ END OF MASTER NOTE ~~~~

Bottom Seal Astragal:  << Required; or  Not required>>.

Guide Surface Weatherstrip:  << Angled brush seal;  Vinyl seal; or  Not required>>.

Lintel Brush Seal:  << Interior;  Exterior;  Interior and exterior; or  Not required>>.

Interior Hood Baffle:  << Required; or  Not required>>.

Side Guides, Channels:  Constructed of << steel;  304 stainless steel; or  aluminum>> with members fully bolted together.

Finish:  << Mill finish;  Clear anodized;  Bronze anodized;  Black anodized;  Powder coat;  Stainless No.4 satin;  Hot-dip galvanized;  Galvanized, primed black;  PowderGuard Textured powder coat;  PowderGuard Zinc powder coat; or  Stainless No.2B mill>>.

Color:  << Match curtain;  Black; or  _____>>.

Brackets:  << Steel; or  304 stainless steel>> to support counterbalance and curtain.

Finish:  << Hot-dip galvanized;  Galvanized, primed black;  Stainless No.4 satin;  Powder coat;  PowderGuard Premium powder coat; _____ color;  PowderGuard Textured powder coat; _____ color; or  PowderGuard Zinc powder coat; _____ color>>.

Color:  << Match curtain; or  _____>>.

Counterbalance:  Helical torsion spring type, housed in steel tube or pipe barrel and supporting curtain with deflection limited to << 0.03 inch per foot (1:400 mm/sec)>> of span. Adjustable spring tension required.

Hood:  << Not required;  24 gauge, 0.028 inch stainless steel (0.63 mm stainless steel);  24 gauge, 0.028 inch galvanized steel (0.63 mm galvanized steel);  22 gauge, 0.025 inch B&S aluminum (0.64 mm B&S aluminum); or  _____ gauge, _____ inch (_____ mm)>>.

Finish:  << Galvanized, primed to match curtain;  Hot-dip galvanized;  Clear anodized;  Standard polyester base coat; gray color;  Bronze anodized;  Standard polyester base coat; tan color;  Mill finish;  Standard polyester base coat; white color;  Powder coat;  _____   color;  Standard polyester base coat; brown color;  Stainless No. 4 satin;  Gray-primed galvanized G90 (Z275);  Black anodized; or  PowderGuard Premium powder coat; _____ color>>.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up available up to 8 by 8 feet (2438 by 2438 mm). Chain hoist over 8 by 8 feet (2438 by 2438 mm).
Bracket-mounted crank available up to 12 feet 4 inches (3759 mm) in height or 10 feet 4 inches (3149 mm) in width.
Wall crank not available if between-jamb mounted; available up to 16 feet 4 inches (4978 mm) in width or 14 feet 4 inches (4369 mm) in height.
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Push-up;  Chain hoist;  Bracket-mounted crank; or  Wall-mounted crank>>.

Motor Operation:  << Not required; or  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction at minimum 8 inches (203 mm), maximum 1 foot (305 mm) per second>>.

Manual Override:  << Hoist;  Hoist and jackshaft release; or  Jackshaft release>>.

Timer to Close:  << Not required; or  Automatic closing controlled by adjustable hold-open time delay>>.

Operation Supply Voltage:  << 115/208/230V 1-phase 60Hz;  208/230/460V 3 phase 60Hz;  575V 3-phase 60Hz;  220V 1-phase 50Hz;  220V 3-phase 50Hz; or  400V 3-phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  << Wall-mounted; or  Front of hood>>, << right-hand side; or  left-hand side>>.

Motor Enclosure:  << Not required;  NEMA 4-12 watertight and oiltight;  NEMA 4X corrosion-resistant;  Totally enclosed, fan-cooled motor; or  Totally enclosed, non-ventilated>>.

Obstruction Safety Detection:  Constant contact button only.

Obstruction Safety Detection:

Primary, Monitored:

Photo Eyes:  << Reflective photo-eyes;  Thru-beam photo-eyes; or  NEMA 4X photo-eyes>>.

Light Curtain:  << 3 feet (0.91 m);  6 feet (1.83 m); or  _____ feet (_____ m)>>.

Safety Edge:  << 2-wire reversing safety edge;  Wireless reversing safety edge; or  ______>>.

Secondary, Non-Monitored:

Photo Eyes:  << Reflective photo-eyes;  Thru-beam photo-eyes; or  _____>>.

Light Curtain:  << 3 feet (0.9 m);  6 feet (1.8 m); or  _____ feet (_____ m)>>.

Safety Edge:  << 2-wire reversing safety edge; or  ______>>.

Wayne Dalton; Model 800 Rolling Service Door.

Slats:  Interlocking, roll-formed, metal slats.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 40 feet (12.19 m). 
Maximum height and width are independent of each other and may vary based on slat profile, material, and gauge. For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 40 feet (12.19 m). 
Maximum height and width are independent of each other and may vary based on slat profile, material, and gauge. For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _______>>.

Mounting:  << Surface-mounted on side indicated on drawings; or  Between jamb>>.

Opening Speed:  Door to operate at variable speed up to << 6 to 12 inches (152 to 305 mm)>> per second.

Closing Speed:  Door to operate at variable speed up to << 6 to 12 inches (152 to 305 mm)>> per second.

Operation Cycles:  Capable of operating for minimum << 20,000;  50,000;  100,000; or  200,000>> cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door Only:  Two years.

Door and Operator Warranty:  Three years or 20,000 cycles, whichever comes first.

- ~~~~~ MASTER NOTE ~~~~~~
Maximum door widths for available wind load ratings:
20 psf: 40 feet (12,192 mm)
31 psf: 33 feet (10,058 mm)
37 psf: 28 feet (8534 mm)
44 psf:  10 feet (3048 mm)
47 psf: 22 feet (6705 mm); impact rated available up to 16 feet (4877 mm)
50 psf: 20 feet (6096 mm); impact rated available
53 psf: 19 feet (5791 mm); impact rated available
55 psf: 22 feet (6705 mm); impact rated available
56 psf: 19 feet (5791 mm); impact rated available
~~~ END OF MASTER NOTE ~~~~

Wind Load:  Design door assembly to withstand ultimate static pressure load of << 20 psf (0.96 kPa);  31 psf (1.48 kPa);  37 psf (1.77 kPa);  44 psf (2.11 kPa);  47 psf (2.25 kPa);  50 psf (2.39 kPa);  53 psf (2.54 kPa);  55 psf (2.63 kPa); or  56 psf (2.68 kPa)>> in accordance with <<ASTM E330/E330M>>.

Wind Load:  Not required.

- ~~~~~ MASTER NOTE ~~~~~~
Impact Rating Requirements:
Dade County: 65 psf (3.1 kPa)
FBC HVHZ Level E: 65 psf (3.1 kPa)
FBC HVHZ: 68.9 psf (3.3 kPa)
~~~ END OF MASTER NOTE ~~~~

Impact Rating:  Available with certification for large missile impact resistance (<<FBC TAS 201>> and <<FBC TAS 202>>), with design pressure of << _____ psf (_____ Pa)>> at << _____ feet (_____ mm)>> wide.<<  This design is approved by the Florida Building Commission, FL # _____ .;    This design is approved by Miami Dade NOA.; or  None - N/A>>

Curtain Material:  << Galvanized steel;  Aluminum; or  Stainless steel>>. Staking lock system locks end of each slat from lateral movement.

- ~~~~~ MASTER NOTE ~~~~~~
Galvanized Steel: 22 gauge for doors up to 15 feet 4 inches (4674 mm) wide, 20 gauge for 18 feet 4 inches (5588 mm) to 25 feet 4 inches (7722 mm) wide, 18 gauge for over 25 feet 4 inches (7722 mm) to 30 feet 4 inches (9245 mm).
~~~ END OF MASTER NOTE ~~~~

Galvanized Steel:  << 18 gauge, 0.0516 inch (1.31 mm);  20 gauge, 0.0396 inch (1.01 mm);  22 gauge, 0.0336 inch (0.86 mm); or  _____ gauge, _____ inch (_____ mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Powder coat available in approximately 200 colors; custom color match options available.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Galvanized, primed white;  Galvanized, primed gray;  Galvanized, primed beige;  Galvanized, primed brown;  Galvanized G90 (Z275);  Galvanized G90 (Z275), primed gray; or  Powder coat; _____ color>>.

Aluminum:  << 0.050 inch (1.27 mm); or  _____ inch (_____ mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Powder coat available in approximately 200 colors; custom color match options available.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Mill finish;  Clear anodized;  Bronze anodized; or  Powder coat;  _____ color>>.

Stainless Steel:  << 20 gauge, 0.0359 inch (0.91 mm); or  _____ gauge, _____ inch (_____ mm)>>.

Finish:  << No.2B mill; or  No.4 satin>>.

- ~~~~~ MASTER NOTE ~~~~~~
No.4 3-inch curved-faced single crown slat.

- 1) 22 gauge galvanized steel.
2) 20 gauge galvanized steel.
3) 18 gauge galvanized steel.
4) 16 gauge galvanized steel.
5) 22 gauge stainless steel.
6) 20 gauge stainless steel.
7) 18 gauge stainless steel.
8) 14 gauge aluminum. No.14 3-inch flat-faced slat. 1) 22 gauge galvanized steel.
2) 20 gauge galvanized steel.
3) 18 gauge galvanized steel.
4) 16 gauge galvanized steel.
5) 22 gauge stainless steel.
6) 20 gauge stainless steel.
7) 18 gauge stainless steel.
8) 16 gauge aluminum.
9) 14 gauge aluminum.
~~~ END OF MASTER NOTE ~~~~

Slat Profile:  << No.4, 3-inch (76 mm) curved slat;  No.14, 3-inch (76 mm) flat slat;  F-265; or  FE-265>>.

- ~~~~~ MASTER NOTE ~~~~~~
Fenestrated slats only available on No.14 slat profile.
Ventilated, or fenestrated, flat-faced slat has 5-inch by 3/4-inch rectangular holes spaced evenly across slat, or has rectangular holes spaced evenly across slat in size selected. Offers 16.5 to 17 percent open area over slat, with a maximum of 20 ventilated slats per curtain. Maximum wind load rating is 20 psf. Available in 22-, 20-, or 18-gauge steel or stainless steel, with or without vision lites.
Secur-Vent perforated slats provide optimal security and ventilation. Perforated with 1/16 inch diameter holes, offering 20 to 25 percent open area over slat. Available for doors up to to 22 feet wide by 20 feet high in 20-gauge steel or stainless steel.
~~~ END OF MASTER NOTE ~~~~

Fenestrated Slats:  << Not required;  Secur-Vent perforated slats;  3/4 by 5 inch (19 by 127 mm) holes;  5/8 by 3 inches (16 by 76 mm);  3/4 by 5 inch (19 by 127 mm) holes with Plexiglas covers; or  1 by 10 inches (25 by 254 mm)>>.

Bottom Bar:  << 6063-T6 aluminum;  Galvanized steel; or  304 Stainless steel>>.

Profile:  Double angle.

Finish:  << Mill finish;  Clear anodized;  Bronze anodized;  Black anodized;  Hot-dip galvanized;  Powder coat;  Stainless No.4 satin;  PowderGuard Textured powder coat;  PowderGuard Zinc powder coat; or  Stainless No.2B mill>>.

Color:  << Match curtain;  Black; or  _____>>.

Locking Options to Include:  << None;  Slide lock;  Thumbturn;  Cylinder; or  Best cylinder>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>>.

Lock Access:  << Coil side; or  Opposite coil side>>.

Weatherstripping and Seals:

- ~~~~~ MASTER NOTE ~~~~~~
Bottom seal astragal comes standard.
~~~ END OF MASTER NOTE ~~~~

Bottom Seal Astragal:  << Required; or  Not required>>.

Guide Surface Weatherstrip:  << Angled brush seal;  Vinyl seal; or  Not required>>.

Lintel Brush Seal:  << Interior;  Exterior;  Interior and exterior; or  Not required>>.

Interior Hood Baffle:  << Required; or  Not required>>.

Side Guides, Channels:  Constructed of << steel;  304 stainless steel; or  aluminum>> with members fully bolted together.

Finish:  << Galvanized, primed black;  Hot-dip galvanized;  Mill finish;  Clear anodized;  Black anodized;  Bronze anodized;  Powder coat;  Stainless No.4 satin;  PowderGuard Textured powder coat;  PowderGuard Zinc powder coat; or  Stainless No.2B mill>>.

Color:  << Match curtain;  Black; or  _____>>.

Brackets:  << Steel; or  304 Stainless steel>> to support counterbalance and curtain.

Finish:  << Galvanized, primed black;  Hot-dip galvanized;  Mill finish;  Clear anodized;  Black anodized;  Bronze anodized;  Powder coat;  Stainless No.4 satin;  Stainless No.2B mill;  PowderGuard Textured powder coat; or  PowerGuard Zinc powder coat>>.

Color:  << Match curtain; or  _____>>.

Counterbalance:  Helical torsion spring type, housed in steel tube or pipe barrel and supporting curtain with deflection limited to << 0.03 inch per foot (1:400 mm/sec)>> of span. Adjustable spring tension required.

Hood:  << Not required;  24 gauge, 0.028 inch stainless steel (0.63 mm stainless steel);  24 gauge, 0.028 inch galvanized steel (0.63 mm galvanized steel);  22 gauge, 0.025 inch B&S aluminum (0.64 mm B&S aluminum); or  _____ gauge, _____ inch (_____ mm)>>.

Finish:  << Galvanized, primed to match curtain;  Hot-dip galvanized;  Clear anodized;  Bronze anodized;  Mill finish;  Powder coat;  _____   color; or  Stainless No.4 satin>>.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up available up to 8 by 8 feet (2438 by 2438 mm). Chain hoist over 8 by 8 feet (2438 by 2438 mm).
Bracket-mounted crank available up to 12 feet 4 inches (3759 mm) in height or 10 feet 4 inches (3149 mm) in width.
Wall crank not available if between-jamb mounted; available up to 16 feet 4 inches (4978 mm) in width or 14 feet 4 inches (4369 mm) in height.
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Push-up;  Chain hoist;  Bracket-mounted crank; or  Wall-mounted crank>>.

Motor Operation:  << Not required; or  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction at minimum 8 inches (203 mm), maximum 1 foot (305 mm) per second>>.

Manual Override:  << Hoist;  Hoist and jackshaft release; or  Jackshaft release>>.

Timer to Close:  << Not required; or  Automatic closing controlled by adjustable hold-open time delay>>.

Operation Supply Voltage:  << 115/208/230V 1-phase 60Hz;  208/230/460V 3 phase 60Hz;  575V 3-phase 60Hz;  220V 1-phase 50Hz;  220V 3-phase 50Hz; or  400V 3-phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  << Wall-mounted; or  Front of hood>>, << right-hand side; or  left-hand side>>.

Motor Enclosure:  << Not required;  NEMA 4-12 watertight and oiltight;  NEMA 4X corrosion-resistant;  Totally enclosed, fan-cooled motor; or  Totally enclosed, non-ventilated>>.

Obstruction Safety Detection:  Constant contact button only.

Obstruction Safety Detection:

Primary, Monitored:

Photo Eyes:  << Reflective photo-eyes;  Thru-beam photo-eyes; or  NEMA 4X photo-eyes>>.

Light Curtain:  << 3 feet (0.91 m);  6 feet (1.83 m); or  _____ feet (_____ m)>>.

Safety Edge:  << 2-wire reversing safety edge;  Wireless reversing safety edge; or  ______>>.

Secondary, Non-Monitored:

Photo Eyes:  << Reflective photo-eyes;  Thru-beam photo-eyes; or  _____>>.

Light Curtain:  << 3 feet (0.9 m);  6 feet (1.8 m); or  _____ feet (_____ m)>>.

Safety Edge:  << 2-wire reversing safety edge; or  ______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Model 800 HC is a springless door system with direct drive motor and 25 times the typical 20,000 operating cycles of standard door operators.
~~~ END OF MASTER NOTE ~~~~

Wayne Dalton; Model 800 HC Springless High Cycle Rolling Service Door.

Slats:  Interlocking, roll-formed, metal slats.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 20 feet (6.1 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 20 feet (6.1 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _______>>.

Mounting:  Surface mounted on side indicated on drawings.

Opening Speed:  Door to operate at variable speed up to << 8 to 12 inches (203 to 305 mm)>> per second.

Closing Speed:  Door to operate at variable speed up to << 8 to 12 inches (203 to 305 mm)>> per second.

Operation Cycles:  Capable of operating for minimum 500,000 cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Limited Warranty:  Three years or 500,000 cycles, whichever comes first.

- ~~~~~ MASTER NOTE ~~~~~~
Maximum door widths for available wind load ratings:
20 psf: 20 feet (6096 mm)
31 psf: 33 feet (10,058 mm); impact rated available up to 22 feet
37 psf: 28 feet (8534 mm); impact rated available up to 22 feet 
44 psf: 10 feet (3048 mm)
47 psf: 22 feet (6705 mm); impact rated available up to 22 feet
53 psf: 19 feet (5791 mm); impact rated available 
55 psf: 22 feet (6705 mm); impact rated available  
~~~ END OF MASTER NOTE ~~~~

Wind Load:  Design door assembly to withstand ultimate static pressure load of << 20 psf (0.96 kPa);  31 psf (1.48 kPa);  37 psf (1.77 kPa);  44 psf (2.11 kPa);  47 psf (2.25 kPa);  50 psf (2.39 kPa);  53 psf (2.54 kPa);  55 psf (2.63 kPa); or  56 psf (2.68 kPa)>> in accordance with <<ASTM E330/E330M>>.

Wind Load:  Not required.

- ~~~~~ MASTER NOTE ~~~~~~
Impact Rating Requirements:
Dade County: 65 psf (3.1 kPa)
FBC HVHZ Level E: 65 psf (3.1 kPa)
FBC HVHZ: 68.9 psf (3.3 kPa)
~~~ END OF MASTER NOTE ~~~~

Impact Rating:  Available with certification for large missile impact resistance (<<FBC TAS 201>> and <<FBC TAS 202>>), with design pressure of << _____ psf (_____ Pa)>> at << _____ feet (_____ mm)>> wide.<<  This design is approved by the Florida Building Commission, FL # _____ .;    This design is approved by Miami Dade NOA.; or  None - N/A>>

Impact Rating:  Not required.

Curtain Material:  << Galvanized steel;  Aluminum; or  Stainless steel>>. Staking lock system locks end of each slat from lateral movement.

Galvanized Steel:  << 18 gauge, 0.0516 inch (1.31 mm);  20 gauge, 0.0396 inch (1.01 mm);  22 gauge, 0.0336 inch (0.86 mm); or  _____ gauge, _____ inch (_____ mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Powder coat available in approximately 200 colors; custom color match options available.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Galvanized, primed white;  Galvanized, primed gray;  Galvanized, primed beige;  Galvanized, primed brown;  Galvanized G90 (Z275);  Galvanized G90 (Z275), primed gray; or  Powder coat; _____ color>>.

Aluminum:  << 0.040 inch (1.02 mm); or  _____ inch (_____ mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Powder coat available in approximately 200 colors; custom color match options available.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Mill finish;  Clear anodized;  Black anodized;  Bronze anodized; or  Powder coat; _____ color>>.

Stainless Steel:  << 20 gauge, 0.0359 inch (0.91 mm);  22 gauge, 0.0299 inch (0.76 mm); or  _____ gauge, _____ inch (_____ mm)>>.

Finish:  << No.2B mill; or  No.4 satin>>.

- ~~~~~ MASTER NOTE ~~~~~~
No.14 flat-faced slat:
1) 22 gauge galvanized steel.
2) 20 gauge galvanized steel.
3) 18 gauge galvanized steel.
4) 22 gauge stainless steel.
5) 20 gauge stainless steel.
6) 16 gauge (.050 inch ) aluminum.
~~~ END OF MASTER NOTE ~~~~

Slat Profile:  No.14, << 3-inch (76 mm)>> tall flat slat.

- ~~~~~ MASTER NOTE ~~~~~~
Ventilated, fenestrated flat-faced slat has 5-inch by 3/4-inch (127 by 19.1 mm) rectangular holes spaced evenly across slat, or per spec. Offers 16.5 to 17 percent open area over slat, with a maximum of 20 ventilated slats per curtain. Maximum wind load rating is 20 psf. Available in 22-, 20-, or 18-gauge steel or stainless steel, with or without vision lites. Fenestrated slats only available on No.14 slat profile.

Ventilated, fenestrated flat-faced slat has 5-inch by 3/4-inch (127 by 19.1 mm) rectangular holes spaced evenly across slat, or per spec. Offers 16.5 to 17 percent open area over slat, with a maximum of 20 ventilated slats per curtain. Maximum wind load rating is 20 psf. Available in 22-, 20-, or 18-gauge steel or stainless steel, with or without vision lites.

Secur-Vent perforated slats provide optimal security and ventilation. Perforated with 1/16 inch (4.2 mm) diamter holes, offering 20 to 25 percent open area over slat. Available for doors up to 22 feet wide by 20 feet high (6.7 m wide by 6.1 m high) in 20-gauge steel or stainless steel.
~~~ END OF MASTER NOTE ~~~~

Fenestrated Slats:  << Not required;  Secur-Vent perforated slats;  3/4 by 5 inch (19 by 127 mm) holes; or  3/4 by 5 inch (19 by 127 mm) holes with Plexiglas covers>>.

Bottom Bar:  << 6063-T6 aluminum;  Galvanized steel; or  304 Stainless steel>>.

Profile:  Double angle.

Finish:  << Mill finish;  Clear anodized;  Bronze anodized;  Hot-dip galvanized;  Powder coat; or  Stainless No.4 satin>>.

Color:  << Match curtain; or  _____>>.

Weatherstripping and Seals:

Bottom Seal Astragal:  Included.

Interior Guide Seal:  << Required; or  Not required>>.

Exterior Guide Seal:  << Required; or  Not required>>.

Lintel Brush Seal:  << Required; or  Not required>>.

Side Guides, Channels:  Constructed of << steel;  304 stainless steel; or  aluminum>> with members fully bolted together.

Finish:  << Hot-dip galvanized;  Galvanized, primed black;  Powder coat;  PowderGuard Textured powder coat;  Stainless No.4 satin; or  PowderGuard Zinc powder coat>>.

Color:  << Match curtain;  Black; or  _____>>.

Brackets:  Steel to support counterbalance and curtain.

Finish:  << Hot-dip galvanized;  Galvanized, primed black;  Powder coat; or  PowderGuard Textured powder coat>>.

Color:  << Match curtain;  Black; or  _____>>.

Door Roll:  Directly driven, springless roll constructed of steel tube with integral shafts, keyed on drive end and supported by self-aligning, greasable, sealed bearings. Counterbalance device not required.

Hood:  << Not required;  24 gauge, 0.028 inch stainless steel (0.63 mm stainless steel);  24 gauge, 0.028 inch galvanized steel (0.63 mm galvanized steel);  22 gauge, 0.025 inch B&S aluminum (0.64 mm B&S aluminum); or  _____ gauge, _____ inch (_____ mm)>>.

Finish:  << Galvanized, primed to match curtain;  Hot-dip galvanized;  Clear anodized;  Bronze anodized;  Mill finish;  Powder coat; _____ color; or  Stainless No.4 satin>>.

Motor Operation:  Direct drive, integrated gear motor and brake assembly; size as recommended by manufacturer to move door in either direction at minimum << 8 inches (203 mm)>>, maximum << 1 foot (305 mm)>> per second. Operator and drive assembly is factory preassembled and provided with wiring harnesses needed direct from factory.

Manual Override:  Hoist.

Timer to Close:  Time-delay self-close timer and non-resettable cycle counter.

Operation Supply Voltage:  << 208V 1-phase 60Hz;  208V 3-phase 60Hz;  230V 3-phase 60Hz;  575V 3-phase 60Hz;  220 to 240V 1-phase 50Hz; or  440 to 480V 3-phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  Shaft mount.

Obstruction Safety Detection:

Primary, Monitored:

Photo Eyes:  << Reflective photo-eyes;  Thru-beam photo-eyes;  Dual photo-eyes; or  _____>>.

Safety Edge:  << 2-wire reversing safety edge; or  ______>>.

Secondary, Non-Monitored:

Photo Eyes:  << Reflective photo-eyes;  Thru-beam photo-eyes;  Dual photo-eyes; or  _____>>.

Safety Edge:  << 2-wire reversing safety edge; or  ______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Model 800 ADV is a springless door system with direct drive motor with 50 times the operating cycles and more than 3 times the speed.
~~~ END OF MASTER NOTE ~~~~

Wayne Dalton; Model 800 ADV.

Slats:  Interlocking, roll-formed, metal slats.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 30 feet (9.14 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 20 feet (6.1 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _______>>.

Mounting:  Surface mounted on side indicated on drawings.

Opening Speed:  Door to operate at variable speed up to << 24 inches (610 mm)>> per second.

Closing Speed:  Door to operate at variable speed up to << 12 inches (305 mm)>> per second.

Operation Cycles:  Capable of operating for minimum one million cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Limited Warranty:  Five years or one million cycles, whichever comes first.

- ~~~~~ MASTER NOTE ~~~~~~
Maximum door widths for available wind load ratings:
20 psf: 20 feet (6.1 m)
31 psf: 33 feet (10.1 m); impact rated available up to 22 feet
37 psf: 28 feet (8.5 m); impact rated available up to 22 feet 
44 psf: 10 feet (3.1 m)
47 psf: 22 feet (6.7 m); impact rated available up to 22 feet
53 psf: 19 feet (5.8 m); impact rated available 
55 psf: 22 feet (6.7 m); impact rated available 
~~~ END OF MASTER NOTE ~~~~

Wind Load:  Design door assembly to withstand ultimate static pressure load of << 20 psf (0.96 kPa);  31 psf (1.48 kPa);  37 psf (1.77 kPa);  44 psf (2.11 kPa);  47 psf (2.25 kPa);  50 psf (2.39 kPa);  53 psf (2.54 kPa);  55 psf (2.63 kPa); or  56 psf (2.68 kPa)>> in accordance with <<ASTM E330/E330M>>.

Wind Load:  Not required.

- ~~~~~ MASTER NOTE ~~~~~~
Impact Rating Requirements:
Dade County: 47 psf (2.25 kPa) and 55 psf (2.63 kPa)
FBC HVHZ: 47 psf (2.25 kPa) and 55 psf (2.63 kPa)
~~~ END OF MASTER NOTE ~~~~

Impact Rating:  Available with certification for large missile impact resistance (<<FBC TAS 201>> and <<FBC TAS 202>>), with design pressure of << _____ psf (_____ Pa)>> at << _____ feet (_____ mm)>> wide.<<   This design is approved by the Florida Building Commission,;    This design is approved by Miami Dade NOA,; or  None - N/A>> << FL # _____;  TDI # _____; or  None - N/A>>.

Impact Rating:  Not required.

Curtain Material:  << Galvanized steel; or  Aluminum>>. Staking lock system locks end of each slat from lateral movement.

Galvanized Steel:  << 22 gauge, 0.037 inch (0.853 mm); or  _____ gauge, _____ inch (_____ mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Hardened powder coat finish, with four times the hardness of standard powder coat, is available in standard colors of white, tan, and gray, or select from approximately 200 RAL colors for an additional charge.
~~~ END OF MASTER NOTE ~~~~

Finish:  Hardened powder coat; << white;  tan;  gray; or  _______>> color.

Aluminum:  << 0.040 inch (1.02 mm); or  _____ inch (_____ mm)>>.

Finish:  << Mill finish;  Clear anodized;  Black anodized;  Bronze anodized; or  Hardened powder coat; _______ color>>.

- ~~~~~ MASTER NOTE ~~~~~~
Curved Profile type C-187, 1.87-inch tall slat, for doors up to 15 feet 4 inches (4.67 m) wide, fabricated of:
1) 22 gauge powder-coated steel.
2) 20 gauge powder-coated steel.
3) 18 gauge powder-coated steel.
4) 22 gauge stainless steel.
5) 20 gauge stainless steel.
6) .040 inch (1 mm) aluminum. Curved Profile type C-275, 2.75-inch tall slat, for doors up to 20 feet (6.1 m) wide, fabricated of:
1) 22 gauge powder-coated steel.
2) 20 gauge powder-coated steel.
3) 18 gauge powder-coated steel.
4) 22 gauge stainless steel.
5) 20 gauge stainless steel.
6) .050 inch (1.29 mm) aluminum. Flat Profile type F-265, 2.65-inch tall slat, for doors up to 20 feet (6.1 m) wide, fabricated of: 1) 22 gauge powder-coated steel.
2) 20 gauge powder-coated steel.
3) 18 gauge powder-coated steel.
4) 22 gauge stainless steel.
5) 20 gauge stainless steel.
6) .050 inch (1.29 mm) aluminum.
~~~ END OF MASTER NOTE ~~~~

Slat Profile:  No.14, << 3-inch (76 mm)>> tall flat slat.

Fenestrated Slats:  << Not required;  3/4 by 5 inch (19 by 127 mm) holes; or  3/4 by 5 inch (19 by 127 mm) holes with Plexiglas covers>>.

Bottom Bar:  << 6063-T6 aluminum;  Galvanized steel; or  304 Stainless steel>>.

Profile:  Double angle.

Finish:  << Mill finish;  Clear anodized;  Bronze anodized;  Hot-dip galvanized;  Powder coat; or  Stainless No.4 satin>>.

Color:  << Match curtain;  Black; or  _______>>.

Weatherstripping and Seals:

Bottom Sensing Edge Seal:  Included.

Interior Guide Seal:  << Required; or  Not required>>.

Exterior Guide Seal:  << Required; or  Not required>>.

Lintel Brush Seal:  << Required; or  Not required>>.

Side Guides, Channels:  Constructed of << steel; or  304 stainless steel>> with members fully bolted together.

Finish:  << Hot-dip galvanized;  Galvanized, primed black;  Powder coat; or  Stainless No.4 satin>>.

Color:  << Match curtain;  Black; or  _______>>.

Brackets:  Steel to support counterbalance and curtain.

Finish:  << Hot-dip galvanized;  Galvanized, primed black; or  Powder coat>>.

Color:  << Match curtain; or  _____>>.

Door Roll:  Directly driven, springless roll constructed of steel tube with integral shafts, keyed on drive end and supported by self-aligning, greasable, sealed bearings. Counterbalance device not required.

Hood:  << Not required;  24 gauge, 0.024 inch stainless steel (0.61 mm stainless steel);  24 gauge, 0.028 inch galvanized steel (0.63 mm galvanized steel);  22 gauge, 0.025 inch B&S aluminum (0.64 mm B&S aluminum); or  _____ gauge, _____ inch (_____ mm)>>.

Finish:  << Galvanized, primed to match curtain;  Hot-dip galvanized;  Clear anodized;  Bronze anodized;  Mill finish;  Powder coat; _____ color; or  Stainless No.4 satin>>.

Motor Operation:  Direct drive, integrated gear motor and brake assembly; size as recommended by manufacturer to move door in either direction at minimum << 8 inches (203 mm)>>, maximum << 1 foot (305 mm)>> per second. Operator and drive assembly is factory preassembled and provided with wiring harnesses needed direct from factory.

Manual Override:  Hoist.

Timer to Close:  Time-delay self-close timer and non-resettable cycle counter.

Operation Supply Voltage:  << 208V 1-phase 60Hz;  208V 3-phase 60Hz;  575V 3-phase 60Hz;  220V 1-phase 50Hz;  230V 3-phase 50Hz; or  460V 3-phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  Shaft mount.

- ~~~~~ MASTER NOTE ~~~~~~
Model 800 ADV comes standard with photo eyes, sensing edge, and built-in brake mechanism.
~~~ END OF MASTER NOTE ~~~~

Obstruction Safety Detection:

Primary, Monitored:  Wireless reversing safety edge.

Secondary, Non-Monitored:  << Reflective photo-eyes;  Dual reflective photo-eyes;  Thru-beam photo-eyes; or  Dual thru-beam photo-eyes>>.

Control Panel:  Provide electronic variable frequency drive controller with microprocessor self-diagnostics. LCD readout indicates door action, alarm conditions, and fault conditions. Enclosure is NEMA 4X-rated. Control system is UL-certified. Junction box is IP67-rated.

- ~~~~~ MASTER NOTE ~~~~~~
Model 625 insulated door is available with standard speed operator.
~~~ END OF MASTER NOTE ~~~~

Wayne Dalton; Model 800C Insulated Rolling Service Door.

Slats:  Insulated, interlocking, roll-formed, metal slats.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 42 feet (12.8 m). 
Maximum height and width are independent of each other and may vary based on slat profile, material, and gauge. For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 30 feet (9.1 m). 
Maximum height and width are independent of each other and may vary based on slat profile, material, and gauge. For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _______>>.

Mounting:  << Surface-mounted on side indicated on drawings; or  Between jamb>>.

Opening Speed:  Door to operate at variable speed up to << 6 to 12 inches (152 to 305 mm)>> per second.

Closing Speed:  Door to operate at variable speed up to << 6 to 12 inches (152 to 305 mm)>> per second.

Operation Cycles:  Capable of operating for minimum << 20,000;  50,000;  100,000; or  200,000>> cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door Only:  Two years.

Door and Operator Warranty:  Three years or 20,000 cycles, whichever comes first.

- ~~~~~ MASTER NOTE ~~~~~~
Maximum door widths for available wind load ratings:
20 psf: 42 feet (12.8 m)
31 psf: 33 feet (10.1 m); impact rated available up to 16 feet
37 psf: 28 feet (8.5 m); impact rated available up to 16 feet 
44 psf: 10 feet (3.1 m)
47 psf: 22 feet (6.7 m); impact rated available up to 16 feet
50 psf: 20 feet (6.1 m)
53 psf: 19 feet (5.8 m) 
55 psf: 22 feet (6.7 m); impact rated available 
56 psf: 19 feet (5.8 m)
~~~ END OF MASTER NOTE ~~~~

Wind Load:  Design door assembly to withstand ultimate static pressure load of << 20 psf (0.96 kPa);  31 psf (1.48 kPa);  37 psf (1.77 kPa);  44 psf (2.11 kPa);  47 psf (2.25 kPa);  50 psf (2.39 kPa);  53 psf (2.54 kPa);  55 psf (2.63 kPa); or  56 psf (2.68 kPa)>> in accordance with <<ASTM E330/E330M>>.

Wind Load:  Not required.

- ~~~~~ MASTER NOTE ~~~~~~
Impact Rating Requirements:
Dade County:  47 psf (2.3 kPa) and 55 psf (2.6 kPa).
FBC HVHZ:  47 psf (2.3 kPa) and 55 psf (2.6 kPa).
~~~ END OF MASTER NOTE ~~~~

Impact Rating:  Available with certification for large missile impact resistance (<<FBC TAS 201>> and <<FBC TAS 202>>), with design pressure of << _____ psf (_____ Pa)>> at << _____ feet (_____ mm)>> wide.<<  This design is approved by the Florida Building Commission, FL # _____ .;    This design is approved by Miami Dade NOA.;    Texas Department of Insurance.; or  None - N/A>>

Impact Rating:  Not required.

Curtain Material:  Staking lock system locks end of each slat from lateral movement.

Front Slat:

Galvanized Steel:  << 18 gauge, 0.0516 inch (1.31 mm);  20 gauge, 0.0396 inch (1.01 mm);  22 gauge, 0.0336 inch (0.86 mm);  24 gauge, 0.0278 inch (0.63 mm); or  ____ gauge, _____ inch (_____ mm)>>.

Aluminum:  << 0.040 inch (1.02 mm); or  _____ inch (_____ mm)>>.

Stainless Steel:  << 18 gauge, 0.05 inch (1.27 mm);  20 gauge, 0.0359 inch (0.91 mm);  22 gauge, 0.0299 inch (0.76 mm); or  _____ gauge, _____ inch (_____ mm)>>.

Back Slat:

Galvanized Steel:  << 24 gauge, 0.0278 inch (0.63 mm);  22 gauge, 0.0336 inch (0.86 mm); or  ____ gauge, _____ inch (_____ mm)>>.

Aluminum:  << 0.024 inch (0.6 mm);  0.040 inch (1.02 mm); or  _____ inch (_____ mm)>>.

Stainless Steel:  << 24 gauge, 0.239 inch (0.61 mm); or  _____ gauge, _____ inch (_____ mm)>>.

Slat Profile:  No.34, << 3-inch (76 mm)>> tall flat slat.

Slat R-Value:  << R-value (RSI-value):  7.7 (1.35)>>.

Installed System Sound Rating:  STC-21 in accordance with <<ASTM E90>>.

Air Infiltration:  Meets <<ASHRAE Std 90.1 I-P>> and <<ICC (IECC)-2018>> C402.4.3 air leakage of fenestration, maximum 1.00 cfm/sq ft.

Vision Lites:  << Not required;  3/4 by 5 inches (19 by 127 mm) with Plexiglas covers; or  1 by 10 inches (25 by 254 mm) with Plexiglas covers>>.

- ~~~~~ MASTER NOTE ~~~~~~
Powder coat available in approximately 200 colors; custom color match options available.
~~~ END OF MASTER NOTE ~~~~

Galvanized Steel Finish:  << Primed white;  Primed gray;  Primed beige;  Primed brown;  G90 (Z275);  G90 (Z275), primed gray; or  Powder coat; _____ color>>.

- ~~~~~ MASTER NOTE ~~~~~~
Powder coat available in approximately 200 colors; custom color match options available.
~~~ END OF MASTER NOTE ~~~~

Aluminum Finish:  << Mill finish;  Clear anodized;  Bronze anodized;  Black anodized; or  Powder coat; _____ color>>.

Stainless Steel Finish:  << No.2B mill; or  No.4 satin>>.

Bottom Bar:  << 6063-T6 aluminum;  Galvanized steel; or  304 Stainless steel>>.

Profile:  Double angle.

Finish:  << Mill finish;  Clear anodized;  Bronze anodized;  Hot-dip galvanized;  Galvanized, primed black;  Powder coat; or  Stainless No.4 satin>>.

Color:  << Match curtain;  Black; or  _______>>.

Locking Options to Include:  << None;  Slide lock;  Thumbturn;  Cylinder; or  Best cylinder>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>>.

Lock Access:  << Coil side; or  Opposite coil side>>.

Weatherstripping and Seals:

Bottom Seal Astragal:  Included.

Exterior Guide Seal:  << Required; or  Not required>>.

Interior Guide Seal:  << Required; or  Not required>>.

Internal Hood Seal:  Included.

Lintel Brush Seal:  << Required; or  Not required>>.

Air Infiltration:  Meets <<ASHRAE Std 90.1 I-P>> and <<ICC (IECC)-2018>> C402.4.3 air leakage of fenestration, maximum 1.00 cfm/sq ft.

Air infiltration perimeter seal package includes guide cover, guide cap, dual brush exterior guide seal, << 4-inch (101.6 mm)>> finned lintel brush seal, and vinyl bottom seal.

Side Guides, Channels:  Constructed of << steel;  304 stainless steel; or  aluminum>> with members fully bolted together.

Finish:  << Galvanized, primed black;  Hot-dip galvanized;  Mill finish;  Clear anodized;  Bronze anodized;  Powder coat; or  Stainless No.4 satin>>.

Color:  << Match curtain;  Black; or  _______>>.

Brackets:  << Steel; or  304 Stainless steel>> to support counterbalance and curtain.

Finish:  << Galvanized, primed black;  Hot-dip galvanized;  Powder coat; or  Stainless No.4 satin>>.

Color:  << Match curtain; or  _______>>.

Counterbalance:  Helical torsion spring type, housed in steel tube or pipe barrel and supporting curtain with deflection limited to << 0.03 inch per foot (1:400 mm/sec)>> of span. Adjustable spring tension required.

Hood:  << Not required;  24 gauge, 0.024 inch stainless steel (0.61 mm stainless steel);  24 gauge, 0.028 inch galvanized steel (0.63 mm galvanized steel);  22 gauge, 0.025 inch B&S aluminum (0.64 mm B&S aluminum); or  _____ gauge, _____ inch (_____ mm)>>.

Finish:  << Galvanized, primed to match curtain;  Hot-dip galvanized;  Clear anodized;  Bronze anodized;  Mill finish;  Powder coat; _____ color; or  Stainless No. 4 satin>>.

Manual Operation:  << Chain hoist; or  Crank>>.

Motor Operation:  << Not required; or  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction at minimum 8 inches (203 mm), maximum 1 foot (305 mm) per second>>.

Manual Override:  << Hoist;  Hoist and jackshaft release; or  Jackshaft release>>.

Timer to Close:  << Not required; or  Automatic closing controlled by adjustable hold-open time delay>>.

Operation Supply Voltage:  << 115/208/230V 1 phase 60Hz;  208/230/460V 3 phase 60Hz;  575V 3 phase 60Hz;  220V 1 phase 50Hz;  220V 3 phase 50Hz; or  400V 3 phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  << Wall-mounted; or  Front of hood>>, << right-hand side; or  left-hand side>>.

Motor Enclosure:  << Not required;  NEMA 4-12 watertight and oiltight;  NEMA 4X corrosion-resistant;  Totally enclosed, fan-cooled motor; or  Totally enclosed, non-ventilated>>.

Obstruction Safety Detection:  Constant contact button only.

Obstruction Safety Detection:

Primary, Monitored:  << Reflective photo-eyes;  Thru-beam photo-eyes;  NEMA 4X photo-eyes;  2-wire reversing safety edge;  Wireless reversing safety edge;  3-foot (0.9 m) light curtain; or  6-foot (1.8 m) light curtain>>.

Secondary, Non-Monitored:  << Reflective photo-eyes;  Thru-beam photo-eyes;  2-wire reversing safety edge;  3-foot (0.9 m) light curtain; or  6-foot (1.8 m) light curtain>>.

- ~~~~~ MASTER NOTE ~~~~~~
Model 800C HC is a springless door system with direct drive motor and 25 times the operating cycles.
~~~ END OF MASTER NOTE ~~~~

Wayne Dalton; Model 800C HC Service Door.

Slats:  Insulated, interlocking, roll-formed, metal slats.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 20 feet (6.1 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 20 feet (6.1 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _______>>.

Mounting:  Surface mounted on side indicated on drawings.

Opening Speed:  Door to operate at variable speed up to << 8 to 12 inches (203 to 305 mm)>> per second.

Closing Speed:  Door to operate at variable speed up to << 8 to 12 inches (203 to 305 mm)>> per second.

Operation Cycles:  Capable of operating for minimum 500,000 cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Limited Warranty:  Three years or 500,000 cycles, whichever comes first.

- ~~~~~ MASTER NOTE ~~~~~~
Maximum door widths for available wind load ratings:
20 psf: 20 feet (6096 mm)
31 psf: 33 feet (10,058 mm); impact rated available up to 22 feet
37 psf: 28 feet (8534 mm); impact rated available up to 22 feet 
44 psf: 10 feet (3048 mm)
47 psf: 22 feet (6705 mm); impact rated available up to 22 feet
53 psf: 19 feet (5791 mm); impact rated available 
55 psf: 22 feet (6705 mm); impact rated available 
~~~ END OF MASTER NOTE ~~~~

Wind Load:  Design door assembly to withstand ultimate static pressure load of << 20 psf (0.96 kPa);  31 psf (1.48 kPa);  37 psf (1.77 kPa);  44 psf (2.11 kPa);  47 psf (2.25 kPa);  50 psf (2.39 kPa);  53 psf (2.54 kPa);  55 psf (2.63 kPa); or  56 psf (2.68 kPa)>> in accordance with <<ASTM E330/E330M>>.

Wind Load:  Not required.

- ~~~~~ MASTER NOTE ~~~~~~
Impact Rating Requirements:
Dade County: 47 psf (2.3 kPa) and 55 psf (2.63 kPa)
FBC HVHZ: 47 psf (2.3 kPa) and 55 psf (2.63 kPa)
~~~ END OF MASTER NOTE ~~~~

Impact Rating:  Available with certification for large missile impact resistance (<<FBC TAS 201>> and <<FBC TAS 202>>), with design pressure of << _____ psf (_____ Pa)>> at << _____ feet (_____ mm)>> wide.<<  This design is approved by the Florida Building Commission, FL # _____ .;    This design is approved by Miami Dade NOA,; or  None - N/A>>

Curtain Material:

Galvanized Steel:  << 18 gauge, 0.0516 inch (1.31 mm);  20 gauge, 0.0396 inch (1.01 mm);  22 gauge, 0.0336 inch (0.86 mm);  24 gauge, 0.0278 inch (0.63 mm); or  ____ gauge, _____ inch (_____ mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Powder coat available in approximately 200 colors; custom color match options available.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Galvanized, primed white;  Galvanized, primed gray;  Galvanized, primed beige;  Galvanized, primed brown;  Galvanized G90 (Z275);  Galvanized G90 (Z275), primed gray; or  Powder coat; _____ color>>.

Aluminum:  << 0.040 inch (1.02 mm); or  _____ inch (_____ mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Powder coat available in approximately 200 colors; custom color match options available.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Mill finish;  Clear anodized;  Bronze anodized; or  Powder coat; _____ color>>.

Stainless Steel:  << 18 gauge, 0.0478 inch (1.27 mm);  20 gauge, 0.0359 inch (0.91 mm);  22 gauge, 0.0299 inch (0.76 mm); or  _____ gauge, _____ inch (_____ mm)>>.

Finish:  << No.2B mill; or  No.4 satin>>.

Slat Profile:  No.34, << 3-inch (76 mm)>> tall flat slat.

Slat R-Value:  << R-value (RSI-value):  7.7 (1.35)>>.

Installed System Sound Rating:  STC-21 in accordance with <<ASTM E90>>.

Vision Lites:  << Not required;  3/4 by 5 inches (19 by 127 mm) with Plexiglas covers; or  1 by 10 inches (25 by 254 mm) with Plexiglas covers>>.

Bottom Bar:  << 6063-T6 aluminum;  Galvanized steel; or  304 Stainless steel>>.

- ~~~~~ MASTER NOTE ~~~~~~
Double angle available for galvanized steel, stainless steel, and aluminum.
Extruded tube available for aluminum only.
~~~ END OF MASTER NOTE ~~~~

Profile:  Double angle.

Finish:  << Mill finish;  Clear anodized;  Bronze anodized;  Hot-dip galvanized;  Galvanized, primed black;  Powder coat; or  Stainless No.4 satin>>.

Color:  << Match curtain;  Black; or  _______>>.

Weatherstripping and Seals:

Bottom Seal Astragal:  Included.

Exterior Guide Seal:  << Required; or  Not required>>.

Interior Guide Seal:  << Required; or  Not required>>.

Internal Hood Seal:  Included.

Lintel Brush Seal:  << Required; or  Not required>>.

Side Guides, Channels:  Constructed of << steel; or  304 stainless steel>> with members fully bolted together.

Finish:  << Hot-dip galvanized;  Galvanized, primed black;  PowderGuard Premium powder coat;  Stainless No.4 satin;  PowderGuard Textured powder coat;  Powder coat; or  PowderGuard Zinc powder coat>>.

Color:  << Match curtain;  _______; or  Black>>.

Brackets:  Steel to support counterbalance and curtain.

Finish:  << Hot-dip galvanized;  Galvanized, primed black; or  Powder coat>>.

Color:  << Match curtain; or  _____>>.

Door Roll:  Directly driven, springless roll constructed of steel tube with integral shafts, keyed on drive end and supported by self-aligning, greasable, sealed bearings. Counterbalance device not required.

Hood:  << Not required;  24 gauge, 0.024 inch stainless steel (0.61 mm stainless steel);  24 gauge, 0.028 inch galvanized steel (0.63 mm galvanized steel);  22 gauge, 0.025 inch B&S aluminum (0.64 mm B&S aluminum); or  _____ gauge, _____ inch (_____ mm)>>.

Finish:  << Galvanized, primed to match curtain;  Hot-dip galvanized;  Clear anodized;  Bronze anodized;  Mill finish;  Powder coat; _____ color; or  Stainless No.4 satin>>.

Motor Operation:  Direct drive, integrated gear motor and brake assembly; size as recommended by manufacturer to move door in either direction at minimum << 8 inches (203 mm)>>, maximum << 1 foot (305 mm)>> per second. Operator and drive assembly is factory preassembled and provided with wiring harnesses needed direct from factory.

Manual Override:  Hoist.

Timer to Close:  Time-delay self-close timer and non-resettable cycle counter.

Operation Supply Voltage:  << 208V 1-phase 60Hz;  208V 3-phase 60Hz;  230V 3-phase 60Hz;  575V 3-phase 60Hz;  220-240V 1 phase 50Hz; or  440-480V 3 phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  Shaft mount.

Obstruction Safety Detection:

Primary, Monitored:  << Reflective photo-eyes;  Thru-beam photo-eyes;  Dual photo-eyes; or  2-wire reversing safety edge>>.

Secondary, Non-Monitored:  << Reflective photo-eyes;  Thru-beam photo-eyes;  Dual photo-eyes; or  2-wire reversing safety edge>>.

Control Panel:  Electronic controller with microprocessor self-diagnostics. Digital readout indicates door action, alarm conditions, and fault conditions. Enclosure is IP54-rated, NEMA 3 equivalent.

- ~~~~~ MASTER NOTE ~~~~~~
Model 800C ADV door has motor with 50 times the operating cycles and more than 3 times the speed.
~~~ END OF MASTER NOTE ~~~~

Wayne Dalton; Model 800C ADV Service Door.

Slats:  Interlocking, roll-formed, galvanized steel slats.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 20 feet (6.1 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 20 feet (6.1 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _______>>.

Mounting:  Surface mounted on side indicated on drawings.

Opening Speed:  Door to operate at variable speed up to << 24 inches (610 mm)>> per second.

Closing Speed:  Door to operate at variable speed up to << 12 inches (305 mm)>> per second.

Operation Cycles:  Capable of operating for minimum one million cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Limited Warranty:  Five years or one million cycles, whichever comes first.

- ~~~~~ MASTER NOTE ~~~~~~
Wind load offering dependent on gauge and door size.
Maximum door widths for available wind load ratings:
20 psf: 20 feet (6.1 m)
31 psf: 33 feet (10.1 m); impact rated available up to 22 feet
37 psf: 28 feet (8.5 m); impact rated available up to 22 feet 
44 psf: 10 feet (3.1 m)
47 psf: 22 feet (6.7 m); impact rated available up to 22 feet 
53 psf: 19 feet (5.8 m); impact rated available
55 psf: 22 feet (6.7 m); impact rated available 
 Reinforced high-pressure rated:
 50 psf: 30 feet 4 inches (9.25 m)
 57 to 130 psf: 10 to 25 feet (3.05 to 7.62 m); ask manufacturer for exact configuration
~~~ END OF MASTER NOTE ~~~~

Wind Load:  Design door assembly to withstand ultimate static pressure load of << 20 psf (0.96 kPa);  31 psf (1.48 kPa);  37 psf (1.77 kPa);  44 psf (2.11 kPa);  47 psf (2.25 kPa);  50 psf (2.39 kPa);  53 psf (2.54 kPa);  55 psf (2.63 kPa); or  56 psf (2.68 kPa)>> in accordance with <<ASTM E330/E330M>>.

Wind Load:  Not required.

- ~~~~~ MASTER NOTE ~~~~~~
Impact Rating Requirements:
Dade County: 47 psf (2.25 kPa) and 55 psf (2.63 kPa)
FBC HVHZ: 47 psf (2.25 kPa) and 55 psf (2.63 kPa)
~~~ END OF MASTER NOTE ~~~~

Impact Rating:  Available with certification for large missile impact resistance (<<FBC TAS 201>> and <<FBC TAS 202>>), with design pressure of << _____ psf (_____ Pa)>> at << _____ feet (_____ mm)>> wide.<<  This design is approved by the Florida Building Commission, FL # _____ .;    This design is approved by Miami Dade NOA.; or  None - N/A>>

Curtain Material:  << Galvanized steel;  Aluminum; or  Stainless steel>>. Staking lock system locks end of each slat from lateral movement.

Galvanized Steel:  << 18 gauge, 0.0516 inch (1.31 mm);  20 gauge, 0.0396 inch (1.01 mm);  22 gauge, 0.0336 inch (0.86 mm);  24 gauge, 0.0278 inch (0.63 mm); or  ____ gauge, _____ inch (_____ mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Hardened powder coat finish, with four times the hardness of standard powder coat, is available in standard colors of white, beige, and gray, or select from 197 RAL colors for an additional charge.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Galvanized, primed white;  Galvanized, primed gray;  Galvanized, primed beige;  Galvanized, primed brown;  Galvanized G90 (Z275);  Galvanized G90 (Z275), primed gray; or  Powder coat; _____ color>>.

Aluminum:  << 0.040 inch (1.02 mm); or  _____ inch (_____ mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Hardened powder coat finish, with four times the hardness of standard powder coat, is available in standard colors of white, beige, and gray, or select from 197 RAL colors for an additional charge.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Mill finish;  Clear anodized;  Bronze anodized; or  Powder coat; _____ color>>.

Stainless Steel:  << 20 gauge, 0.0359 inch (0.91 mm);  22 gauge, 0.0299 inch (0.76 mm); or  _____ gauge, _____ inch (_____ mm)>>.

Finish:  << No.2B mill; or  No.4 satin>>.

Slat Profile:  No.34, << 3-inch (76 mm)>> tall flat slat.

Slat R-Value:  << R-value (RSI-value):  7.7 (1.35)>>.

Installed System Sound Rating:  STC-21 in accordance with <<ASTM E90>>.

Vision Lites:  << Not required; or  3/4 by 5 inches (19 by 127 mm) with Plexiglas covers>>.

Bottom Bar:  << 6063-T6 aluminum;  Galvanized steel; or  304 stainless steel>>.

Profile:  Double angle.

Finish:  << Mill finish;  Clear anodized;  Bronze anodized;  Hot-dip galvanized;  Galvanized, primed black;  Powder coat; or  Stainless No.4 satin>>.

Color:  << Match curtain; or  _______>>.

Weatherstripping and Seals:

- ~~~~~ MASTER NOTE ~~~~~~
Bottom sensing edge seal comes standard.
~~~ END OF MASTER NOTE ~~~~

Bottom Sensing Edge Seal:  Included.

Exterior Guide Seal:  << Required; or  Not required>>.

Interior Guide Seal:  << Required; or  Not required>>.

Internal Hood Seal:  Included.

Lintel Brush Seal:  << Required; or  Not required>>.

Side Guides, Channels:  Constructed of << steel; or  304 stainless steel>> with members fully bolted together.

Finish:  << Hot-dip galvanized;  Galvanized, primed black;  Stainless No.4 satin; or  Powder coat>>.

Color:  << Match curtain; or  _______>>.

Brackets:  Steel to support counterbalance and curtain.

Finish:  << Hot-dip galvanized;  Galvanized, primed black; or  Powder coat>>.

Color:  << Match curtain; or  _____>>.

Door Roll:  Directly driven, springless roll constructed of steel tube with integral shafts, keyed on drive end and supported by self-aligning, greasable, sealed bearings. Counterbalance device not required.

Hood:  << Not required;  24 gauge, 0.024 inch stainless steel (0.61 mm stainless steel);  24 gauge, 0.028 inch galvanized steel (0.63 mm galvanized steel);  22 gauge, 0.025 inch B&S aluminum (0.64 mm B&S aluminum); or  _____ gauge, _____ inch (_____ mm)>>.

Finish:  << Galvanized, primed to match curtain;  Hot-dip galvanized;  Clear anodized;  Bronze anodized;  Mill finish;  Powder coat; _____ color; or  Stainless No.4 satin>>.

Motor Operation:  Direct drive, integrated gear motor and brake assembly; size as recommended by manufacturer to move door in either direction at minimum << 8 inches (203 mm)>>, maximum << 1 foot (305 mm)>> per second. Operator and drive assembly is factory preassembled and provided with wiring harnesses needed direct from factory.

Manual Override:  Hoist.

Timer to Close:  Time-delay self-close timer and non-resettable cycle counter.

Operation Supply Voltage:  << 208V 1-phase 60Hz;  208V 3-phase 60Hz;  575V 3-phase 60Hz;  220V 1-phase 50Hz;  230V 3-phase 50Hz; or  460V 3-phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  Shaft mount.

- ~~~~~ MASTER NOTE ~~~~~~
626 comes standard with photoelectric sensors, sensing edge, and built-in brake mechanism.
~~~ END OF MASTER NOTE ~~~~

Obstruction Safety Detection:

Primary, Monitored:  Wireless reversing safety edge.

Secondary, Non-Monitored:  << Reflective photo-eyes;  Dual reflective photo-eyes;  Thru-beam photo-eyes; or  Dual thru-beam photo-eyes>>.

Control Panel:  Provide electronic variable frequency drive controller with microprocessor self-diagnostics. LCD readout indicates door action, alarm conditions, and fault conditions. Enclosure is NEMA 4X-rated. Control system is UL-certified. Junction box is IP67-rated.

Upward Coiling Security Shutters

Wayne Dalton; Model 523 Security Shutter.

Slats:  Interlocking, extruded aluminum slats.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 20 feet (6.1 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 14 feet (4.3 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _______>>.

Mounting:  << Surface-mounted on side indicated on drawings; or  Between jamb>>.

Opening Speed:  Door to operate at variable speed up to << 8 inches (203 mm)>> per second.

Closing Speed:  Door to operate at variable speed up to << 12 inches (305 mm)>> per second.

Curtain Material:  << 0.050-inch (1.27 mm)>> thick aluminum.

Fenestrated Slats:  << Not required;  Open fenestration; 1-by-1-inch (25.4 by 25.4 mm) uniformly spaced openings over full curtain;  Partially open fenestration; 1-by-1-inch (25.4 by 25.4 mm) uniformly spaced openings. Height of fenestrated curtain segment as indicated on drawings;  Perforated slats; full curtain; or  Perforated slats; partial curtain. Height of perforated curtain segment as indicated on drawings>>.

Aluminum Finish:  << White;  Almond;  Silver powder coat to match look of clear anodized aluminum;  Bronze powder coat to match look of bronze anodized; or  Powder coat; _____ color>>.

Bottom Bar:  6063-T6 aluminum.

Profile:  << Compact extruded; or  Extruded tube>>.

Finish:  Match curtain.

- ~~~~~ MASTER NOTE ~~~~~~
Slide lock and padlockable slide lock available only with compact extruded bottom bar placed left, right, or at both jambs.
Cylinder and Best cylinder only available in center location on extruded tube bottom bar.
~~~ END OF MASTER NOTE ~~~~

Locking Options to Include:  << None;  Slide lock;  Cylinder; or  Best cylinder>>.

Lock Locations:  << Both jambs;  Left only;  Right only; or  Center>>.

Lock Access:  << Coil side;  Opposite coil side; or  Both sides>>.

Interlock micro switch on locking doors with tube motor operation.

Weatherstripping and Seals:

Bottom Seal Astragal:  Required.

Side Guides, Channels:  Constructed of aluminum extruded channels with continuous PVC wear strips.

Finish:  Match curtain.

Brackets:  Galvanized steel to support counterbalance, curtain, and hood.

Finish:  Match curtain.

Counterbalance:  Helical torsion spring type, housed in steel tube or pipe barrel and supporting curtain with deflection limited to << 0.03 inch per foot (1:400 mm/sec)>> of span. Adjustable spring tension required.

Hood:  Aluminum. Box sized to match manufacturer's recommendation based on door height. Provide with intermediate support brackets as required on drawings.

Finish:  << Match curtain; or  Powder coat; _____ color>>.

Manual Operation:  << Push-up; or  Crank>>.

- ~~~~~ MASTER NOTE ~~~~~~
Motor only available on door width equal to or minimum 3 feet (914 mm) and maximum 14 feet (4.3 m), with height maximum 6 feet (1.8 m).
~~~ END OF MASTER NOTE ~~~~

Motor Operation:  Provide UL-listed electric tube motor; size as recommended by manufacturer to move door in either direction at minimum << 8 inches (203 mm)>>, maximum << 1 foot (305 mm)>> per second.

Manual Override:  << Not required; or  Crank>>.

Operation Supply Voltage:  << 110V; or  240V>>.

Actuation:  Double throw, hard-wired wall switch.

Motor Mounting:  Within counterbalance tube.

Upward Coiling Sheet Doors

Wayne Dalton; Model 770SS Sheet Door.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 10 feet (3 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 10 feet (3 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _______>>.

Mounting:  Surface mounted on side indicated on drawings.

Opening Speed:  Door to operate at variable speed up to << 8 inches (203 mm)>> per second.

Closing Speed:  Door to operate at variable speed up to << 12 inches (305 mm)>> per second.

Operation Cycles:  Capable of operating for minimum 10,000 cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Curtain:  Roll-formed, << 26 gauge, 0.217 inch (0.55 mm)>>, << G30 (Z90)>> galvanized steel sheet in accordance with <<ASTM A653/A653M>>, SS Grade 80. Sections interlocked and permanently seamed together to form continuous curtain. Provided with PVC edge strip stapled on edge of exterior side to minimize steel-to-steel contact, enhance door operation, and minimize curtain nesting and scratching.

Finish:  Rust-inhibitive roll coating process, including << 0.2 mil (0.0051 mm)>> baked-on prime paint and << 0.6 mil (0.015 mm)>> baked-on polyester top coat.

Color:  << White;  Glossy White;  Silhouette Gray;  Buckskin;  Desert Tan;  Garnet Red;  Walnut Brown;  Polar Blue;  Royal Blue;  Teal;  Dark Teal;  Forest Green;  Bronze; or  Matte black>>.

Bottom Bar:  Extruded aluminum reinforced with << 1-1/2-by-1-1/2-inch (38 by 38 mm)>> roll-formed steel angle and provided with flexible PVC bulb-type astragal to ensure consistent seal along floor. Extrusion designed to interlock with door curtain.

Bottom Bar Stops:  << 12-gauge, 0.108-inch (2.75 mm)>> galvanized steel with quick-connect design allowing curtain insertion into universal guide and lock into place with one fastener.

Finish:  Mill finish.

Locking Options to Include:  << None;  Slide lock; or  Slide lock that accepts cylinder or padlock>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>> in bottom bar.

Lock Access:  << Coil side; or  Opposite coil side>>.

Optional Dual Exterior Slide Bolt Curtain Locks:  << Required; or  Not required>>.

Weatherstripping and Seals:

Bottom Seal Astragal:  Included.

Guides:  << 1-5/8 inches (41 mm)>> wide, roll-formed, << 18-gauge, 0.0478-inch (1.21 mm)>> steel guides with UHMW polypropylene rub strips on each edge. Through-hole design allows easy access from front for fastener attachment to concrete, wood, steel, or masonry jambs. Prepunched to accept quick-connect attachment of headplate and bottom bar stops.

Finish:  Galvanized.

Headplates:  << 14-gauge, 0.0747-inch (1.90 mm)>> steel with steel roller bearings to prevent steel-to-steel contact for improved door operations and extended door shaft life. Quick-connect design for quick assembly to guides.

Finish:  Galvanized.

Counterbalance:  Assembly includes stepped design steel rings to ensure tight and uniform curtain wrap, << 3-3/8-inch (86 mm)>> inside diameter springs, and << 1-5/16-inch (35 mm)>> diameter shaft with << 0.065-inch (1.65 mm)>> wall thickness to minimize door deflection.

Manual Operation:  Push-up.

Wayne Dalton; Model 780CD sheet door.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 16 feet (4.9 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 16 feet (4.9 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _______>>.

Mounting:  Surface mounted on side indicated on drawings.

Opening Speed:  Door to operate at variable speed up to << 8 inches (203 mm)>> per second.

Closing Speed:  Door to operate at variable speed up to << 12 inches (305 mm)>> per second.

Operation Cycles:  Capable of operating for minimum 10,000 cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Curtain:  << 26-gauge, 0.217-inch (0.55 mm)>> galvanized steel sheets.

Finish:  Standard polyester base coat; << white;  glossy white;  silhouette gray;  buckskin;  desert tan;  garnet red;  walnut brown;  polar blue;  royal blue;  teal;  dark teal;  forest green;  bronze; or  matte black>> color.

Bottom Bar:  Extruded aluminum reinforced with << 1-1/2- by 2-inch (38 by 51 mm)>> roll-formed steel angle and provided with flexible PVC bulb-type astragal to ensure consistent seal along floor. Extrusion designed to interlock with door curtain.

Bottom Bar Stops:  << 12-gauge, 0.108-inch (2.75 mm)>> galvanized steel with quick-connect design allowing curtain insertion into universal guide and lock into place with one fastener.

Finish:  Mill finish.

Locking Options to Include:  << None;  Slide lock; or  Slide lock that accepts cylinder or padlock>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>> in bottom bar.

Lock Access:  << Coil side; or  Opposite coil side>>.

Optional Dual Exterior Slide Bolt Curtain Locks:  << Required; or  Not required>>.

Weatherstripping and Seals:

Bottom Seal Astragal:  Included.

Guides:  << 3 inches (76 mm)>> wide, roll-formed, << 18-gauge, 0.0478-inch (1.21 mm)>> steel guides with UHMW polypropylene rub strips on each edge. Through-hole design allows easy access from front for fastener attachment to concrete, wood, steel, or masonry jambs. Prepunched to accept quick-connect attachment of bottom bar stops.

Finish:  Galvanized.

Headplates:  << Heavy-duty thick welded steel;  Stamped steel ; or  _____>>mounted directly to wall to support door shaft and ensure smooth door roll operation.

Finish:  Galvanized.

Counterbalance:  Assembly includes stepped design steel rings with steel roller bearings to ensure tight, uniform curtain wrap, enhanced door operation, and cycle life; factory-lubricated << 3-3/8-inch (86 mm)>> inside diameter springs; and << 1-5/16-inch (35 mm)>> diameter shaft with << 0.065-inch (1.65 mm)>> wall thickness to minimize door deflection. Design allows quick assembly of non-handed chain hoist on either side of door.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up to 84 sq ft (7804 sq mm) or 12 by 7 feet (3658 by 2134 mm). Chain hoist over 84 sq ft (7804 sq m) or 12 by 7 feet (3658 by 2134 mm).
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Push-up; or  Chain hoist>>.

Motor Operation:  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction at minimum << 8 inches (203 mm)>>, maximum << 1 foot (305 mm)>> per second.

Manual Override:  << Hoist;  Hoist and jackshaft release; or  Jackshaft release>>.

Timer to Close:  << Not required; or  Automatic closing controlled by adjustable hold-open time delay>>.

Operation Supply Voltage:  << 115/208/230V 1-phase 60Hz;  208/230/460V 3-phase 60Hz;  575V 3-phase 60Hz;  220V 1-phase 50Hz;  220V 3-phase 50Hz; or  400V 3-phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  << Wall-mounted; or  Front of hood>>, << right-hand side; or  left-hand side>>.

Obstruction Safety Detection:  Infrared sensor mounted in-plane to door curtain.

Reversing Safety Edge:  << Not required;  Door provided with wireless monitored electric safety edge; or  Door provided with wired monitored electric safety edge>>.

Wayne Dalton; Model 790CW Sheet Door.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 10 feet (3 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 10 feet (3 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _______>>.

Mounting:  Surface mounted on side indicated on drawings.

Opening Speed:  Door to operate at variable speed up to << 8 inches (203 mm)>> per second.

Closing Speed:  Door to operate at variable speed up to << 12 inches (305 mm)>> per second.

Operation Cycles:  Capable of operating for minimum 10,000 cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

- ~~~~~ MASTER NOTE ~~~~~~
Maximum door widths for available wind load ratings:
22 psf: 19 feet (5.8 m)
26 psf: 17 to 18 feet (5.18 to 5.49 m)
31 psf: 15 to 16 feet (4.57 to 4.88 m)
36 psf: 13 to 14 feet (3.96 to 4.27 m)
41 psf: 8 to 12 feet (2.44 to 3.66 m)
~~~ END OF MASTER NOTE ~~~~

Wind Load:  Design door assembly to withstand ultimate static pressure load of << 22 psf (1.05 kPa);  26 psf (1.25 kPa);  31 psf (1.48 kPa);  36 psf (1.72 kPa); or  41 psf (1.96 kPa)>> in accordance with <<ASTM E330/E330M>>.

Curtain:  Roll-formed, << 26-gauge, 0.217-inch (0.55 mm)>>, << G30 (Z90)>> galvanized steel sheet in accordance with <<ASTM A653/A653M>>, SS Grade 80. Sections interlocked and permanently seamed together to form continuous curtain. Provide with PVC edge strip stapled on edge of exterior side to minimize steel-to-steel contact, enhance door operation, and minimize curtain nesting and scratching.

Wind Locks:  9-gauge ductile iron wind locks on each edge of curtain, spaced in accordance with door size and wind load requirements.

Finish:  Rust-inhibitive roll coating process including << 0.2 mil (0.0051 mm)>> baked-on prime paint and << 0.6 mil (0.015 mm)>> baked-on polyester top coat.

Color:  << White;  Glossy White;  Silhouette Gray;  Buckskin;  Desert Tan;  Garnet Red;  Walnut Brown;  Polar Blue;  Royal Blue;  Teal;  Dark Teal;  Forest Green;  Bronze; or  Matte black>>.

Bottom Bar:  Extruded aluminum reinforced with << 1-1/2-by-2-inch (38 by 51 mm)>> roll-formed steel angle; provide with flexible, PVC bulb-type astragal, ensuring consistent seal along floor. Extrusion designed to interlock with door curtain.

Bottom Bar Stops:  << 12-gauge, 0.108-inch (2.75 mm)>> galvanized steel with quick-connect design, allowing curtain insertion into universal guide and lock into place with one fastener.

Finish:  Mill finish.

Locking Options to Include:  << None;  Slide lock; or  Slide lock that accepts cylinder or padlock>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>> in bottom bar.

Lock Access:  << Coil side; or  Opposite coil side>>.

Optional Dual Exterior Slide Bolt Curtain Locks:  << Required; or  Not required>>.

Weatherstripping and Seals:

Bottom Seal Astragal:  Included.

Guides:  << 3 inches (76 mm)>> wide, roll-formed, << 14 gauge, 0.0747 inch (1.90 mm)>>, Grade 50 steel guides with UHMW polypropylene rub strips on each edge. Through-hole design allows easy access from front for fastener attachment to concrete, wood, steel, or masonry jambs. Prepunched to accept quick-connect attachment of bottom bar stops.

Finish:  Galvanized.

Headplates:  << Stamped, 11-gauge steel ; or  Heavy-duty, 0.187-inch (4.76 mm) thick welded steel>>mounted directly to wall to support door shaft and ensure smooth door roll operation.

Finish:  Galvanized.

Counterbalance:  Assembly includes stepped design steel rings with steel roller bearings to ensure tight, uniform curtain wrap, enhanced door operation, and cycle life; factory-lubricated, << 3-3/8-inch (86 mm)>> inside diameter springs; and << 1-5/16-inch (35 mm)>> diameter shaft with << 0.065-inch (1.65 mm)>> wall thickness to minimize door deflection. Design allows quick assembly of non-handed chain hoist on either side of door.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up to 84 sq ft (7804 sq mm) or 12 by 7 feet (3658 by 2134 mm). Chain hoist over 84 sq ft (7804 sq m) or 12 by 7 feet (3658 by 2134 mm).
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Push-up; or  Chain hoist>>.

Motor Operation:  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction at minimum << 8 inches (203 mm)>>, maximum << 1 foot (305 mm)>> per second.

Manual Override:  << Hoist;  Hoist and jackshaft release; or  Jackshaft release>>.

Timer to Close:  << Not required; or  Automatic closing controlled by adjustable hold-open time delay>>.

Operation Supply Voltage:  << 115/208/230V 1-phase 60Hz;  208/230/460V 3-phase 60Hz;  575V 3-phase 60Hz;  220V 1-phase 50Hz;  220V 3-phase 50Hz; or  400V 3-phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  << Wall-mounted; or  Front of hood>>, << right-hand side; or  left-hand side>>.

Obstruction Safety Detection:  Infrared sensor mounted in-plane to door curtain.

Reversing Safety Edge:  << Not required;  Door provided with wireless monitored electric safety edge; or  Door provided with wired monitored electric safety edge>>.

PART 3 EXECUTION

Examination

Verify << existing conditions; and  _______>> meet manufacturer's requirements before starting work.

Verify opening sizes, tolerances, and conditions are acceptable.

Installation

Install in accordance with manufacturer's written instructions.

Use anchorage devices to securely fasten assembly to wall construction and building structure without distortion or stress.

Securely and rigidly brace components suspended from structure.<<  Secure guides to structural members only.;    _____ .; or  None - N/A>>

Fit and align assembly, including hardware. Level and plumb to provide smooth operation.

Coordinate installation of electrical service; see Section << 260583>>.

Install enclosure and perimeter trim.

Tolerances

Maintain dimensional tolerances and alignment with adjacent work.

Maximum Variation from Plumb:  << 1/16 inch (1.6 mm); or  ___ inch (___ mm)>>.

Maximum Variation from Level:  << 1/16 inch (1.6 mm); or  ___ inch (___ mm)>>.

Longitudinal or Diagonal Warp:  Plus or minus << 1/8 inch per 10 feet (3.2 mm per 3 m); or  ___ inch per 10 feet (___ mm per 3 m)>> straight edge.

Adjusting

Adjust operating assemblies for smooth and noiseless operation.

Cleaning

See Section << 017000 - Execution and Closeout Requirements>> for additional requirements.

Clean installed components.

Remove labels and visible markings.

Protection

Protect finishes until completion of project.

Touch up damaged finishes after Date of Substantial Completion.

- END OF SECTION